HEAVY VEHICLE NATIONAL LAW

National Heavy Vehicle Standards (Road Train Non-Standard Coupling Height) Exemption Notice 2021 (No.1)

- Purpose

The purpose of this Notice is to exempt a heavy vehicle to which it applies from specified requirements of the Heavy Vehicle (Vehicle Standards) National Regulation (the National Regulation).

Note: 	This is not an authorisation to access the road network, you may need to comply with the relevant Guidelines/Notices in that jurisdiction or apply for a permit if the masses or loaded dimensions exceed the limits in the Regulations, Guidelines/Notices in that jurisdiction.

- Authorising Provision

This Notice is made under section 61 of the Heavy Vehicle National Law (the National Law) as in force in each participating jurisdiction.

- Title

This Notice may be cited as the National Heavy Vehicle Standards (Road Train Non-standard Coupling Height) Exemption Notice 2021 (No.1).

- Commencement

This Notice commences on 1 December 2021.

- Expiry

This Notice expires on 30 November 2026.

- Definitions

- Unless otherwise stated, words and expressions used in this Notice have the same meanings as those defined in the National Law and the Australian Design Rules.

- In this Notice:

ADR means Australian Design Rules.

Drawbar pivot point means the point of vertical articulation where the drawbar attaches to the trailer body.

Non-Standard coupling height means a coupling height that does not comply with ADR 63/00 Clause 63.5.1.2.

Substantially level means that the vertical articulation of the drawbar does not exceed +/- 5 degrees from horizontal when the unladen combination is parked on level ground.

Vehicle label means a permanent, securely attached label that has information written in bold black letters no less than 10mm high on a yellow background stating the following:

- the vehicle is fitted with a non-standard coupling height; and

- the coupling height is reduced; and

- the height of the coupling; and

- that the vertical articulation of the drawbar must not exceed +/- 5 degrees when parked on level ground.

- Application

- This Notice applies to any unit of a road train that has a non-standard coupling height for either:

- an automatic pin coupling to the rear; or

- a drawbar to the front.

- This Notice applies in all participating jurisdictions.

- A vehicle to which this section applies is an eligible vehicle.

- Exemption from Prescribed Vehicle Standards

- An eligible vehicle that complies with the conditions of this Notice is exempt from the following requirements of the Heavy Vehicle (Vehicle Standards) National regulation:

- Schedule 1 - Vehicle standards relating to ADRs applying to single heavy vehicles

Section 2 as applied to ADR 63/00 Clause 63.5.1.2 - Rear coupling height

- Schedule 3 – Vehicle standards applying only to heavy combinations or particular heavy combinations

Section 29(c) - Attachment of couplings and drawbar eyes on road trains

- Conditions

- An eligible vehicle must be built so that when parked on level ground the coupling or drawbar pivot point:

- is at a height of at least 600 millimetres, but not more than 950 millimetres, from the ground; and

- the drawbar is substantially level when in combination.

- An eligible vehicle fitted with a non-standard automatic pin coupling height must be fitted with a vehicle label on or next to the coupling.

- An eligible vehicle fitted with a non-standard drawbar pivot point height must be fitted with a vehicle label immediately adjacent the drawbar eye.

Peter Austin

Director Vehicle Safety and Performance

National Heavy Vehicle Regulator