HEAVY VEHICLE NATIONAL LAW

Tasmania Class 3 Heavy Vehicles Transporting High-Cube Freight Containers Dimension Exemption Notice 2023 (No.1)

- Purpose

- This Notice grants exemptions from mass and dimension requirements for a class 3 heavy combination that is transporting certain freight container.

- This Notice revokes and replaces the Tasmania Class 3 Heavy Vehicles Transporting High-Cube Freight Containers Dimension Exemption Notice 2019 (No.1).

- Authorising provision

- This Notice is made under the following provision of the Heavy Vehicle National Law (HVNL):

- section 117 – Regulator’s power to exempt category of class 1 or 3 heavy vehicles from compliance with mass or dimension requirement.

- section 23 of Schedule 1 – Regulator’s power to amend or repeal instrument.

- Title

This exemption Notice may be cited as the Tasmania Class 3 Heavy Vehicles Transporting High-Cube Freight Containers Dimension Exemption Notice 2023 (No.1).

- Commencement date

This Notice commences on 14 June 2023.

- Expiry date

This Notice expires on 13 June 2028.

- Definitions

- Unless otherwise stated, words and expressions used in this Notice have the same meanings as those in the HVNL and its regulations.

- Definitions from the National Class 2 B-Double Authorisation Notice apply under this notice.

- In this Notice:

High cube freight container means a freight container constructed to a height of between 2.89m and 3.45m.

- Application

- This exemption notice applies to a Class 3 heavy vehicle that meets all the requirements in this section.

- This exemption notice applies to a heavy vehicle that is:

- a rigid truck; or

- a prime mover towing a semi-trailer; or

- a B-Double.

- This Notice only applies to a vehicle specified in 1) if that vehicle is transporting a high cube freight container.

- This Notice applies in Tasmania.

- A heavy vehicle to which this section applies and that complies with the conditions of this notice is an eligible vehicle.

- Exemption – Prescribed dimension requirements

- An eligible vehicle is exempt from the following dimension requirements under Schedule 6 of the Heavy Vehicle (Mass Dimension and Loading) National Regulation (MDL Regulation):

- section 8 – Height

- An exemption in 1) only applies to the extent that specific conditional dimension limits are provided in section 9 of this notice.

- In relation to a particular vehicle or component, if this notice provides for more than one dimension limit to apply in the same circumstances, the more restrictive dimension limit applies.

- Condition – Dimension limit (height)

An eligible vehicle must not exceed 4.5m in height.

Note:	this is the height of the vehicle, including a high cube freight container.

- Condition – stated areas or routes

- An eligible vehicle complying with the conditions of this notice is authorised to operate on the areas and routes specified in this section.

- An eligible vehicle that is a rigid truck or a combination consisting of a prime mover towing a semitrailer may operate on all roads in Tasmania.

- An eligible vehicle that is a B-double may operate on any routes that it would otherwise be authorized to use under the Tasmanian schedule of the National Class 2 B-Double Authorisation Notice.

- Regardless of the access granted under 2) and 3) an eligible vehicle must not exceed the height clearances specified in the Height Clearance Under Overhead Structures Map.

Note:	The Height Clearance Under Overhead Structures Map is maintained by the Tasmanian Department of State Growth and is published on its website.

- Regardless of any other condition or requirement under this notice, an eligible vehicle traveling on the Tasman Bridge must not exceed 4.3m in height.

- For the purposes of section 119(1)(a) of the HVNL, an area or route specified in this section is a stated area or route to which this notice applies.

- An eligible vehicle operating on an area or route specified in this section must comply with the following conditions as specified for routes and areas set out on that map:

- Road conditions pursuant to section 160 of the HVNL; and

- Travel conditions pursuant to section 161 of the HVNL; and

- Vehicle conditions pursuant to section 162 of the HVNL.

David Hourigan

Chief Regulatory and Policy Standards Officer

National Heavy Vehicle Regulator