HEAVY VEHICLE NATIONAL LAW

Queensland Class 3 B-Double and Road Train (Carriage of Large Indivisible Items) Dimension Exemption Notice 2024 (No.1)

- Purpose

This notice grants exemptions from dimension requirements for a class 3 heavy vehicle that is a b-double or a road train combination transporting a large indivisible item.

- Authorising provision

- This notice is made under the following provision of the Heavy Vehicle National Law (HVNL):

- section 117 – Regulator’s power to exempt category of class 1 or 3 heavy vehicles from compliance with mass or dimension requirement.

- Title

This exemption notice may be cited as the Queensland Class 3 B-Double and Road Train (Carriage of Large Indivisible Items) Dimension Exemption Notice 2024 (No.1).

- Commencement date

This notice commences on 10 February 2024.

- Expiry date

This notice expires on 9 February 2029.

- Definitions

- Unless otherwise stated, words and expressions used in this notice have the same meanings as those in the HVNL and its regulations.

- In this notice, definitions from the following notices apply:

- National Class 2 B-Double Authorisation Notice, as amended from time to time; and

- National Class 2 Road Train Authorisation Notice, as amended from time to time.

- In this Notice:

Guide means the Queensland Class 3 B-Double and Road Train (Carriage of Indivisible Items) Dimension Exemption Notice Operator’s Guide, published by the National Heavy Vehicle Regulator.

- Application

- This exemption notice applies to a class 3 heavy vehicle that meets all the requirements in this section.

- This exemption notice applies to a heavy vehicle that is:

- a B-double; or

- a type 1 or type 2 road train.

- This notice only applies to a heavy vehicle specified in 1) if that vehicle is transporting a large indivisible item.

- This notice applies in Queensland.

- A heavy vehicle to which this section applies and that complies with the conditions of this notice is an eligible vehicle.

- Exemption – Prescribed dimension requirements

- An eligible vehicle is exempt from the following dimension requirements under Schedule 6 of the Heavy Vehicle (Mass Dimension and Loading) National Regulation (MDL Regulation):

- section 7 – Width

- section 8 – Height

- An exemption in 1) only applies to the extent that specific conditional dimension limits are provided in section 9 and 10 of this notice.

- The exemptions under this section only apply to the extent that any dimension exceeding prescribed dimension requirements is caused by a large indivisible item.

- In relation to a particular vehicle or component, if this notice provides for more than one dimension limit to apply in the same circumstances, the more restrictive dimension limit applies.

- Condition – Dimension limits (heights for given widths)

- This section applies to an eligible vehicle operating on any road under this notice except those set out in section 10.

- An eligible vehicle that is up to 2.5m in width must not exceed 4.8m in height.

- An eligible vehicle that is up to 3.2m in width must not exceed 4.6m in height.

- For this section, the width of a heavy vehicle must be measured in the same manner as in section 8 of the Heavy Vehicle (Vehicle Standards) National Regulation.

- Conditions – Dimension limits (heights and widths for road trains on the Barkley Highway)

- This section applies to eligible vehicles that are road trains operating on the Barkley Highway between the Queensland and Northern Territory border and Camooweal.

- A type 1 road train must not exceed 3.5m in width or 4.85m in height.

- A type 2 road train must not exceed 3.2m in width or 4.85m in height.

- For this section, the width of a heavy vehicle must be measured in the same manner as in section 8 of the Heavy Vehicle (Vehicle Standards) National Regulation.

- Condition – loading large indivisible items

- An eligible vehicle may carry one or more large indivisible items up to the dimension limits set out in section 9 and 10.

- On an eligible vehicle complying with section 9(2) a large indivisible item may be loaded on any trailer in the combination.

- On an eligible vehicle complying with section 9(3) a combination described in column 1 of Table 1 may only load a large indivisible item on a trailer or truck as described in column 2.

Table 1: Loading locations for large individual items

| Column 1 | Column 2 |
| --- | --- |
| Combination | Allowed loading locations for large indivisible items |
| B-double combination | Any trailer of the combination |
| Type 1 road train combination with a prime mover |  |
| Type 2 road train combination with a prime mover | First and second trailer |
| Type 2 road train combination with a rigid truck | Truck and first trailer |

- Conditions – Drive group restriction

- An eligible vehicle must not have a drive axle group with more than two axles in contact with the ground.

- In this section drive axle group means an axle group that has one or more driving axles.

- Conditions – Quad axle group restriction

An eligible vehicle that is a road train or a B-double may not have a quad axle group on any its components when operating on a State controlled road.

- Application of Schedule 8 of the MDL Regulation

- The following sections of Schedule 8 of the MDL Regulation apply to eligible vehicles as though the vehicle were a class 1 vehicle pursuant to section 39 of the MDL Regulation:

- Part 1 Division 1 General conditions

- Part 1 Division 2 Load-carrying vehicles:

  - section 13 – Carrying goods in addition to a large indivisible item

- Condition – Stated areas or routes

- An eligible vehicle complying with the conditions of this notice is authorised to operate on the areas and routes specified in this section.

- An eligible vehicle described in column 1 of Table 2 may operate on the networks specified in column 2.

Table 2: Networks

| Column 1 | Column 2 |
| --- | --- |
| Eligible vehicle | Network |
| B-double combination | Type 1 Road Train and PBS 3A Route Type 2 Road Train and PBS 4A Route |
| Type 1 road train |  |
| Type 2 road train | Type 2 Road Train and PBS 4A Route |

- For the purposes of section 119(1)(a) of the HVNL, an area or route specified in this section is a stated area or route to which this notice applies.

- In addition to any conditions set out in the networks under table 2, an eligible vehicle must comply with any relevant conditions in the Queensland Access Conditions Guide.

Note:	The Queensland Access Conditions Guide – Route and operational access conditions, as published by the Queensland Government Department of Transport and Main Roads.

- An eligible vehicle must not exceed 90kph.

- An eligible vehicle operating on an area or route specified in this section must comply with the following conditions:

- Road conditions pursuant to section 160 of the HVNL; and

- Travel conditions pursuant to section 161 of the HVNL; and

- Vehicle conditions pursuant to section 162 of the HVNL.

Peter Austin

Director, Policy Implementation

National Heavy Vehicle Regulator