HEAVY VEHICLE NATIONAL LAW

- Queensland Class 3 Heavy Unloaded Trailer and Converter Dolly Dimension Exemption Notice 2024 (No.1)

Purpose

- The purpose of this notice is to exempt class 3 heavy vehicles that are semitrailers towed behind road train rated prime movers from the prescribed height dimension requirement in section 8(d) of Schedule 6 of the Heavy Vehicle (Mass, Dimension and Loading) National Regulation 2013 (the National Regulation).

Authorising Provision

- This notice is made under section 117 of the Heavy Vehicle National Law as in force in each state and territory (HVNL).

Title

- This Notice may be cited as the Queensland Class 3 Heavy Unloaded Trailer and Converter Dolly Dimension Exemption Notice 2024 (No.1).

Commencement

- This Notice commences on 10 February 2024.

Expiry

- This Notice expires on 9 February 2029.

Definitions

- Unless otherwise stated, words and expressions used in this notice have the same meanings as those defined in the HVNL.

- In this notice—

‘Type 1 road train’ means a road train consisting of a prime mover hauling unit towing two trailers when the combination length is not longer than 36.5m.

Note: B-triple and AB-triple combinations up to 36.5m are categorised as a Type 1 road train.

‘Type 2 road train’ means a road train consisting of—

  - a rigid truck hauling unit towing two trailers when the combination length is not longer than 47.5m; or

  - a road train using a prime mover hauling unit towing three or four trailers when the combination length is not longer than 53.5m.

Note:		An AB-triple combination longer than 36.5m and up to a length of 44.0m is categorised as a Type 2 road train

Application

- This Notice applies only in Queensland.

- This Notice applies to a combination comprising—

- a road train rated prime mover; and

- at least one semitrailer attached to the prime mover; and

- up to two unloaded trailers, and up to two converter dollies carried on a trailer mentioned in (b).

Exemption from prescribed dimension limit

- This Notice exempts a semitrailer to which it applies from a dimension limit specified in section 8(d) of Schedule 6 of the National Regulation.

Conditions

- A vehicle to which this notice applies may use a route or area mentioned in clause 16 or 17.

- A vehicle to which this notice applies may operate to a maximum height of 4.6 metres

- The drawbar of a dolly being carried must be secured to a trailer loaded above it to minimise rear overhang.

- A warning flag must be attached to the extreme end of the dolly drawbar.

- If travelling at night, in addition to the flag mentioned in clause 13, a rear facing delineator must be attached to the extreme end of the dolly drawbar and a warning light is attached to the rear of vehicle in the combination which has contact with the road.

Note: Specifications and requirements for flags, delineators and warning lights can be located in the National Regulation.

Condition - Routes or areas

- A Type 1 road train to which this notice applies may use the Type 1 and Type 2 road train routes and areas set out in Appendix 1.

- A Type 2 road train to which this notice applies may use the Type 2 road train routes and areas set out in Appendix 1.

Peter Austin

Director, Policy Implementation

National Heavy Vehicle Regulator

Appendix 1

Type 1 road train routes and areas: Multi-combination Routes in Queensland

Type 2 road train routes and areas: Multi-combination Routes in Queensland

Note:	the routes and areas set out in this Appendix are maintained by TMR and are published on its website.