LICENCE AREA PLAN

SYDNEY RADIO

#### DECEMBER 1999

LICENCE AREA PLAN – SYDNEY RADIO - DECEMBER 1999

# DETERMINATION

The Australian Broadcasting Authority hereby makes this plan for broadcasting services in the Sydney area of New South Wales, under subsection 26(1) of the Broadcasting Services Act 1992.

Seven national radio broadcasting services, ten commercial radio broadcasting services, eight community and three open narrowcasting radio services are to be available in the area described at Attachment 1.1 to this Determination with the use of the broadcasting services bands. The characteristics, including technical specifications, of the services that are to be available in the area described at Attachment 1.1 are set out in Schedule One and Attachments 1.2 - 1.29 to this Determination.

One commercial radio broadcasting service is to be available in the area described at Attachment 2.1 to this Determination with the use of the broadcasting services bands.  The characteristics, including technical specifications, of the service that is to be available in the area described at Attachment 2.1 are set out in Schedule Two and Attachment 2.2 - 2.4 to this Determination.

One commercial and one community radio broadcasting service is to be available in the area described at Attachment 3.1 to this Determination with the use of the broadcasting services bands.  The characteristics, including technical specifications, of the service that is to be available in the area described at Attachment 3.1 are set out in Schedule Three and Attachments 3.2 – 3.3 to this Determination.

One community radio broadcasting service is to be available in the area described at Attachment 4.1 to this Determination with the use of the broadcasting services bands.  The characteristics, including technical specifications, of the service that is to be available in the area described at Attachment 4.1 are set out in Schedule Four and Attachments 4.2 – 4.3 to this Determination.

One community radio broadcasting service is to be available in the area described at Attachment 5.1 to this Determination with the use of the broadcasting services bands.  The characteristics, including technical specifications, of the service that is to be available in the area described at Attachment 5.1 are set out in Schedule Five and Attachment 5.2 to this Determination.

One community radio broadcasting service is to be available in the area described at Attachment 6.1 to this Determination with the use of the broadcasting services bands.  The characteristics, including technical specifications, of the service that is to be available in the area described at Attachment 6.1 are set out in Schedule Six and Attachment 6.2 to this Determination.

One community radio broadcasting service is to be available in the area described at Attachment 7.1 to this Determination with the use of the broadcasting services bands.  The characteristics, including technical specifications, of the service that is to be available in the area described at Attachment 7.1 are set out in Schedule Seven and Attachments 7.2 – 7.3 to this Determination.

One community radio broadcasting service is to be available in the area described at Attachment 8.1 to this Determination with the use of the broadcasting services bands.  The characteristics, including technical specifications, of the service that is to be available in the area described at Attachment 8.1 are set out in Schedule Eight and Attachment 8.2 to this Determination.

One community radio broadcasting service is to be available in the area described at Attachment 9.1 to this Determination with the use of the broadcasting services bands.  The characteristics, including technical specifications, of the service that is to be available in the area described at Attachment 9.1 are set out in Schedule Nine and Attachment 9.2 to this Determination.

One community radio broadcasting service is to be available in the area described at Attachment 10.1 to this Determination with the use of the broadcasting services bands.  The characteristics, including technical specifications, of the service that is to be available in the area described at Attachment 10.1 are set out in Schedule Ten and Attachments 10.2 – 10.5 to this Determination.

One community radio broadcasting service is to be available in the area described at Attachment 11.1 to this Determination with the use of the broadcasting services bands.  The characteristics, including technical specifications, of the service that is to be available in the area described at Attachment 11.1 are set out in Schedule Eleven and Attachment 11.2 to this Determination.

One community radio broadcasting service is to be available in the area described at Attachment 12.1 to this Determination with the use of the broadcasting services bands.  The characteristics, including technical specifications, of the service that is to be available in the area described at Attachment 12.1 are set out in Schedule Twelve and Attachment 12.2 to this Determination.

One community radio broadcasting service is to be available in the area described at Attachment 13.1 to this Determination with the use of the broadcasting services bands.  The characteristics, including technical specifications, of the service that is to be available in the area described at Attachment 13.1 are set out in Schedule Thirteen and Attachment 13.2 to this Determination.

One community radio broadcasting service is to be available in the area described at Attachment 14.1 to this Determination with the use of the broadcasting services bands.  The characteristics, including technical specifications, of the service that is to be available in the area described at Attachment 14.1 are set out in Schedule Fourteen and Attachment 14.2 to this Determination.

One community radio broadcasting service is to be available in the area described at Attachment 15.1 to this Determination with the use of the broadcasting services bands.  The characteristics, including technical specifications, of the service that is to be available in the area described at Attachment 15.1 are set out in Schedule Fifteen and Attachment 15.2 to this Determination.

One community radio broadcasting service is to be available in the area described at Attachment 16.1 to this Determination with the use of the broadcasting services bands.  The characteristics, including technical specifications, of the service that is to be available in the area described at Attachment 16.1 are set out in Schedule Sixteen and Attachment 16.2 to this Determination.

One community radio broadcasting service is to be available in the area described at Attachment 17.1 to this Determination with the use of the broadcasting services bands.  The characteristics, including technical specifications, of the service that is to be available in the area described at Attachment 17.1 are set out in Schedule Seventeen and Attachment 17.2 to this Determination.

One community radio broadcasting service is to be available in the area described at Attachment 18.1 to this Determination with the use of the broadcasting services bands.  The characteristics, including technical specifications, of the service that is to be available in the area described at Attachment 18.1 are set out in Schedule Eighteen and Attachments 18.2 – 18.3 to this Determination.

(Determined by the Australian Broadcasting Authority on 16 December 1999)

SCHEDULES

Each of the Schedules sets out:

- the category of each of the services that are to be available in that area (column one).

- The status of the service (column two) does not form part of the licence area plan decision and is included for reference only.

- The frequency for each transmitter (column three) does not form part of the licence area plan decision and is included for reference only.

- the service licence number which has been allocated to the service, if appropriate (column four);  N/A = Not Applicable, as the Broadcasting Services Act 1992 only requires service licences for Commercial and Community services.

- the number of transmitters required to transmit the service,

- the transmitter specification number which has been allocated to each transmitter (column five),

- the identifying number of the attachment where the technical specifications for each transmitter are described (column six).  (Each set of technical specifications referred to in the Schedules is an attachment to this determination).

- the general area served by each transmitter (column seven).

## Status Column

Licensed	:	service is already licensed

Available	:	new licence available for allocation

Proposed	:	transmitter for existing broadcasting service licence

Alternative	:	alternative channel proposed for licensed service

Reserved	:	capacity reserved for a national or community service in accordance with Ministerial notification.

The information in the status column is provided as additional information to the licence area plan, and is accurate as at the date of publication.

##### Frequency Column

- The frequency shown is the centre frequency of the channel.

The information in the frequency column is provided as additional information to the licence area plan, and is accurate as at the date of publication.

SCHEDULE ONE

Licence Area Plan: Sydney Radio - December 1999

Licence Area: SYDNEY RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| National | Licensed Reserved | 576 kHz | N/A | TS4004001 | 1.2 | Sydney |
| National | Licensed Reserved | 630 kHz | N/A | TS2214001 | 1.3 | Sydney |
| National | Licensed Reserved | 702 kHz | N/A | TS4003001 | 1.4 | Sydney |
| National | Licensed Reserved | 1107 kHz | N/A | TS10004053 | 1.5 | Sydney |
| National | Licensed Reserved | 92.9 MHz | N/A | TS3006001 | 1.6 | Sydney |
| National | Licensed Reserved | 97.7 MHz | N/A | TS6917901 | 1.7 | Sydney |
| National | Licensed Reserved | 105.7 MHz | N/A | TS3005001 | 1.8 | Sydney |
| Commercial | Licensed | 873 kHz | SL4099 | TS4099001 | 1.9 | Sydney |
| Commercial | Licensed | 954 kHz | SL4102 | TS4102001 | 1.10 | Sydney |
| Commercial | Licensed | 1017 kHz | SL4100 | TS4100001 | 1.11 | Sydney |
| Commercial | Licensed | 1170 kHz | SL4098 | TS4098001 | 1.12 | Sydney |
| Commercial | Licensed | 1269 kHz | SL4101 | TS4101001 | 1.13 | Sydney |
| Commercial | Available | 95.3 MHz | SL1150481 | TS10009024 | 1.14 | Sydney |
| Commercial | Available | 96.9 MHz | SL1150104 | TS6229001 | 1.15 | Sydney |
| Commercial | Licensed | 104.1 MHz | SL3032 | TS3032004 | 1.16 | Sydney |
| Commercial | Licensed | 104.9 MHz | SL3033 | TS3033004 | 1.17 | Sydney |
| Commercial | Licensed | 106.5 MHz | SL4103 | TS4103001 | 1.18 | Sydney |
| Community | Licensed | 1224 kHz | SL5157 | TS5157002 | 1.19 | Sydney |
| Community | Available | 92.1 MHz | SL1150110 | TS4825001 | 1.20 | Sydney |
| Community | Available | 93.7 MHz | SL1150111 | TS6228001 | 1.21 | Sydney |
| Community | Available | 94.5 MHz | SL1150109 | TS4826001 | 1.22 | Sydney |
| Community | Licensed | 98.5 MHz | SL2290 | TS10002290 | 1.23 | Sydney |
| Community | Licensed | 102.5 MHz | SL3047 | TS10004307 | 1.24 | Sydney |
| Community | Licensed | 103.2 MHz | SL3044 | TS3044001 | 1.25 | Sydney |
| Community | Licensed | 107.3 MHz | SL3059 | TS3059001 | 1.26 | Sydney |
| Open Narrowcasting | Available | 1386 kHz | N/A | TS10008323 | 1.27 | Sydney |
| Open Narrowcasting | Available | 1476 kHz | N/A | TS10005070 | 1.28 | Penrith |
| Open Narrowcasting | Available | 1539 kHz | N/A | TS8671001 | 1.29 | Sydney |

SCHEDULE TWO

Licence Area Plan: Sydney Radio - December 1999

Licence Area: WESTERN SUBURBS RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Commercial | Licensed | 101.7 MHz | SL4104 | TS4104001 | 2.2 | Sydney |
|  | Proposed | 88.3 MHz | SL4104 | TS10009796 | 2.3 | Camden |
|  | Proposed | 99.1 MHz | SL4104 | TS10009797 | 2.4 | Richmond |

SCHEDULE THREE

Licence Area Plan: Sydney Radio - December 1999

Licence Area: CAMPBELLTOWN RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Commercial | Available | 91.3 MHz | SL1150490 | TS10009041 | 3.2 | Campbelltown |
| Community | Available | 100.3 MHz | SL1150150 | TS10008284 | 3.3 | Campbelltown |

SCHEDULE FOUR

Licence Area Plan: Sydney Radio - December 1999

Licence Area: BANKSTOWN RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Community | Licensed | 88.7 MHz | SL3043 | TS3043001 | 4.2 | Bankstown |
|  | Alternative | 100.9 MHz | SL3043 | TS10009148 | 4.3 | Bankstown |

SCHEDULE FIVE

Licence Area Plan: Sydney Radio - December 1999

Licence Area: BLACKTOWN RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Community | Available | 99.9 MHz | SL1150152 | TS10008287 | 5.2 | Blacktown |

SCHEDULE SIX

Licence Area Plan: Sydney Radio - December 1999

Licence Area: BURWOOD RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Community | Licensed | 88.1 MHz | SL3054 | TS3054001 | 6.2 | Burwood |

SCHEDULE SEVEN

Licence Area Plan: Sydney Radio - December 1999

Licence Area: CHATSWOOD RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Community | Licensed | 91.5 MHz | SL3052 | TS3052001 | 7.2 | Chatswood |
|  | Alternative | 99.3 MHz | SL3052 | TS10008273 | 7.3 | Chatswood |

SCHEDULE EIGHT

Licence Area Plan: Sydney Radio - December 1999

Licence Area: HORNSBY RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Community | Available | 100.1 MHz | SL1150154 | TS10008289 | 8.2 | Hornsby |

SCHEDULE NINE

Licence Area Plan: Sydney Radio - December 1999

Licence Area: LIVERPOOL RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Community | Licensed | 89.3 MHz | SL3046 | TS3046001 | 9.2 | Liverpool |

SCHEDULE TEN

Licence Area Plan: Sydney Radio - December 1999

Licence Area: MANLY RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Community | Licensed | 93.7 MHz | SL3049 | TS3049001 | 10.2 | Manly North |
|  | Alternative | 88.7 MHz | SL3049 | TS10008216 | 10.3 | Manly North |
|  | Licensed | 92.1 MHz | SL3049 | TS3049002 | 10.4 | Manly South |
|  | Alternative | 90.3 MHz | SL3049 | TS10008215 | 10.5 | Manly South |

SCHEDULE ELEVEN

Licence Area Plan: Sydney Radio - December 1999

Licence Area: NARWEE RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Community | Licensed | 90.1 MHz | SL3050 | TS3050001 | 11.2 | Narwee |

SCHEDULE TWELVE

Licence Area Plan: Sydney Radio - December 1999

Licence Area: PARRAMATTA RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Community | Licensed | 90.5 MHz | SL1251 | TS1251001 | 12.2 | Parramatta |

SCHEDULE THIRTEEN

Licence Area Plan: Sydney Radio - December 1999

Licence Area: PENRITH RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Community | Available | 100.7 MHz | SL1150151 | TS10008286 | 13.2 | Penrith |

SCHEDULE FOURTEEN

Licence Area Plan: Sydney Radio - December 1999

Licence Area: RYDE RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Community | Licensed | 88.5 MHz | SL3057 | TS3057001 | 14.2 | Ryde |

SCHEDULE FIFTEEN

Licence Area Plan: Sydney Radio - December 1999

Licence Area: SUTHERLAND RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Community | Licensed | 99.7 MHz | SL4842 | TS4842001 | 15.2 | Sutherland |

SCHEDULE SIXTEEN

Licence Area Plan: Sydney Radio - December 1999

Licence Area: SYDNEY RA2

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Community | Licensed | 88.9 MHz | SL3058 | TS3058001 | 16.2 | Sydney |

SCHEDULE SEVENTEEN

Licence Area Plan: Sydney Radio - December 1999

Licence Area: WAVERLEY RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Community | Licensed | 89.7 MHz | SL3056 | TS3056001 | 17.2 | Waverley |

SCHEDULE EIGHTEEN

Licence Area Plan: Sydney Radio - December 1999

Licence Area: WINDSOR RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Community | Licensed | 89.7 MHz | SL5943 | TS10002532 | 18.2 | Windsor |
|  | Alternative | 89.9 MHz | SL5943 | TS10008374 | 18.3 | Windsor |

DISCLAIMER:

The availability of frequency capacity in the Sydney region is, to a large extent, based on theoretical interference predictions.  Predictions by their very nature are subject to some degree of uncertainty given limitations of topography and other data.

The Australian Broadcasting Authority (ABA) takes reasonable care in assigning frequencies, but cannot warrant that licensed use of a broadcasting facility will be free from (or will not cause) interference.  Any such interference can vary in extent and can originate within Australia or overseas.

Prospective applicants for licences should obtain independent technical or specialist advice concerning their applications, the operation of broadcasting equipment, and any other matters relevant to the operation of transmitters and services under the licences in question.

- Existing broadcasters should take all reasonable steps to be satisfied that their frequency will operate at the prescribed levels before making changes to its existing technical operating conditions.

- Aspirant broadcasters should take all reasonable steps to be satisfied that their frequency of interest will operate at the prescribed levels before competing for spectrum availability.

No liability is or will be accepted by the Minister for Communications, Information Technology and the Arts, the ABA, the Commonwealth of Australia, or its officers, servants or agents for the licensed use of a broadcasting facility which is not free from (or causes) interference.

Attachment 1.1

LICENCE AREA FOR SERVICE LICENCE NOS : SL4099, SL4102, SL4100, SL4098, SL4101, SL1150481, SL1150104, SL3032, SL3033, SL4103, SL5157, SL1150110, SL1150111, SL1150109, SL2290, SL3047, SL3044, SL3059

COMMERCIAL AND COMMUNITY, SYDNEY RA1

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

Blacktown (C)

Sutherland Shire (A)

Fairfield (C)

Penrith (C)

Bankstown (C)

Campbelltown (C)

Parramatta (C)

Hornsby (A)

Canterbury (C)

Warringah (A)

Liverpool (C)

Baulkham Hills (A)

Randwick (C)

Ku-ring-gai (A)

Ryde (C)

Rockdale (C)

South Sydney (C)

Holroyd (C)

Marrickville (A)

Hurstville (C)

Waverley (A)

Leichhardt (A)

North Sydney (A)

Willoughby (C)

Pittwater (A)

Auburn (A)

Woollahra (A)

Kogarah (A)

Ashfield (A)

Manly (A)

Camden (A)

Drummoyne (A)

Lane Cove (A)

Burwood (A)

Strathfield (A)

Mosman (A)

Sydney (C)

Concord (A)

Hunters Hill (A)

NSW CD 1421502

NSW CD 1231909

NSW CD 1421508

NSW CD 1230410

NSW CD 1231908

NSW CD 1230401

NSW CD 1231204

NSW CD 1421908

NSW CD 1421603

NSW CD 1230208

NSW CD 1421702

NSW CD 1230207

NSW CD 1231305

NSW CD 1421902

NSW CD 1231208

NSW CD 1231905

NSW CD 1230608

NSW CD 1230204

NSW CD 1421803

NSW CD 1230606

NSW CD 1231205

NSW CD 1231913

NSW CD 1231902

NSW CD 1230603

NSW CD 1230510

NSW CD 1231210

NSW CD 1421506

NSW CD 1421909

NSW CD 1421707

NSW CD 1230604

NSW CD 1230309

NSW CD 1421804

NSW CD 1230609

NSW CD 1421704

NSW CD 1230405

NSW CD 1421701

NSW CD 1230611

NSW CD 1421710

NSW CD 1231303

NSW CD 1231910

NSW CD 1231304

NSW CD 1230409

NSW CD 1230503

NSW CD 1421811

NSW CD 1421612

NSW CD 1231310

NSW CD 1230304

NSW CD 1230408

NSW CD 1421906

NSW CD 1231209

NSW CD 1421806

NSW CD 1230302

NSW CD 1421807

NSW CD 1231901

NSW CD 1421706

NSW CD 1421709

NSW CD 1231904

NSW CD 1230501

NSW CD 1231301

NSW CD 1421511

NSW CD 1421609

NSW CD 1421613

NSW CD 1230310

NSW CD 1421907

NSW CD 1421805

NSW CD 1231203

NSW CD 1230607

NSW CD 1231308

NSW CD 1421708

NSW CD 1421607

NSW CD 1421809

NSW CD 1231206

NSW CD 1231907

NSW CD 1230605

NSW CD 1421910

NSW CD 1230206

NSW CD 1231912

NSW CD 1230311

NSW CD 1421610

NSW CD 1421505

NSW CD 1231207

NSW CD 1231202

NSW CD 1231211

NSW CD 1230512

NSW CD 1230307

NSW CD 1421813

NSW CD 1230202

NSW CD 1231306

NSW CD 1230406

NSW CD 1230312

NSW CD 1231307

NSW CD 1230308

NSW CD 1230411

NSW CD 1421912

NSW CD 1230506

NSW CD 1230507

NSW CD 1421602

NSW CD 1230404

NSW CD 1231312

NSW CD 1230303

NSW CD 1230203

NSW CD 1421507

NSW CD 1421510

NSW CD 1230601

NSW CD 1230212

NSW CD 1231906

NSW CD 1421604

NSW CD 1231309

NSW CD 1230602

NSW CD 1421911

NSW CD 1421608

NSW CD 1421605

NSW CD 1231313

NSW CD 1421512

NSW CD 1230504

NSW CD 1230508

NSW CD 1421810

NSW CD 1231201

NSW CD 1421711

NSW CD 1421705

NSW CD 1421808

NSW CD 1230610

NSW CD 1421802

NSW CD 1230402

NSW CD 1230301

NSW CD 1230502

NSW CD 1421903

NSW CD 1231302

NSW CD 1421503

NSW CD 1230205

NSW CD 1230313

NSW CD 1230511

NSW CD 1231903

NSW CD 1421801

NSW CD 1421509

NSW CD 1230505

NSW CD 1231311

NSW CD 1230201

NSW CD 1421703

NSW CD 1230209

NSW CD 1421601

NSW CD 1230210

NSW CD 1421611

NSW CD 1421812

NSW CD 1230314

NSW CD 1421606

NSW CD 1421901

NSW CD 1230306

NSW CD 1230403

NSW CD 1230305

NSW CD 1230509

NSW CD 1421905

NSW CD 1421614

NSW CD 1421504

NSW CD 1230407

NSW CD 1421501

NSW CD 1231911

NSW CD 1230211

Note:

Standard terminology used by the Australian Bureau of Statistics

(C) 	=  	City

(CD) 	=  	Collection District

(A)	= 	NSW Local Government Area (excluding Cities)

Attachment 1.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	National

General Area Served :	Sydney (NSW)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - AM Radio

Specification Number :	TS4004001

Transmitter Site :-

Nominal location :	LIVERPOOL

Australian Map Grid :	Zone	Easting	Northing

Reference	56	304640	6242038

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	MF-AM

Carrier Frequency :	576 kHz

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Elevation | Maximum CMF |
| --- | --- | --- |
| At all angles of azimuth | 0 | 2.69 kV |

Attachment 1.3

LICENCE AREA PLAN : Sydney – December 1999

Category :	National

General Area Served :	Sydney (NSW)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - AM Radio

Specification Number :	TS2214001

Transmitter Site :-

Nominal location :	LIVERPOOL

Australian Map Grid :	Zone	Easting	Northing

Reference	56	304640	6242038

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	MF-AM

Carrier Frequency :	630 kHz

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Elevation | Maximum CMF |
| --- | --- | --- |
| At all angles of azimuth | 0 | 995 V |

Attachment 1.4

LICENCE AREA PLAN : Sydney – December 1999

Category :	National

General Area Served :	Sydney (NSW)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - AM Radio

Specification Number :	TS4003001

Transmitter Site :-

Nominal location :	LIVERPOOL

Australian Map Grid :	Zone	Easting	Northing

Reference	56	304482	6242057

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	MF-AM

Carrier Frequency :	702 kHz

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Elevation | Maximum CMF |
| --- | --- | --- |
| At all angles of azimuth | 0 | 3.11 kV |

Attachment 1.5

LICENCE AREA PLAN : Sydney – December 1999

Category :	National

General Area Served :	Sydney (NSW)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - AM Radio

Specification Number :	TS10004053

Transmitter Site :-

Nominal location :	Salt Marsh HOMEBUSH BAY

Australian Map Grid :	Zone	Easting	Northing

Reference	56	322050	6253750

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	MF-AM

Carrier Frequency :	1107 kHz

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Elevation | Maximum CMF |
| --- | --- | --- |
| At all angles of azimuth | 0 | 920 V |

Attachment 1.6

LICENCE AREA PLAN : Sydney – December 1999

Category :	National

General Area Served :	Sydney (NSW)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS3006001

Transmitter Site :-

Nominal location :	ABC Studios GORE HILL

Australian Map Grid :	Zone	Easting	Northing

Reference	56	331921	6256122

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	92.9 MHz

Polarisation	Mixed

Maximum antenna height	130 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 60 kW |

Attachment 1.7

LICENCE AREA PLAN : Sydney – December 1999

Category :	National

General Area Served :	Sydney (NSW)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS6917901

Transmitter Site :-

Nominal location :	ABC Studios GORE HILL

Australian Map Grid :	Zone	Easting	Northing

Reference	56	331921	6256122

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	97.7 MHz

Polarisation	Mixed

Maximum antenna height	130 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 60 kW |

Attachment 1.8

LICENCE AREA PLAN : Sydney – December 1999

Category :	National

General Area Served :	Sydney (NSW)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS3005001

Transmitter Site :-

Nominal location :	ABC Studios GORE HILL

Australian Map Grid :	Zone	Easting	Northing

Reference	56	331921	6256122

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	105.7 MHz

Polarisation	Mixed

Maximum antenna height	130 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 60 kW |

Attachment 1.9

LICENCE AREA PLAN : Sydney – December 1999

Category :	Commercial

General Area Served :	Sydney (NSW)

Service Licence Number :	SL4099

TECHNICAL SPECIFICATION - AM Radio

Specification Number :	TS4099001

Transmitter Site :-

Nominal location :	HOMEBUSH BAY

Australian Map Grid :	Zone	Easting	Northing

Reference	56	322500	6255602

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	MF-AM

Carrier Frequency :	873 kHz

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Elevation | Maximum CMF |
| --- | --- | --- |
| At all angles of azimuth | 0 | 955 V |

Special Conditions :-

This facility can operate with a CMF not to exceed 1350 V for all angle of azimuth, from one hour after local sunrise, to one hour before local sunset as defined at the transmitter site on the 15th day of each month (day-night switching).

Operation of this facility in accordance with Day/Night Switching can commence after October 2000.

Attachment 1.10

LICENCE AREA PLAN : Sydney – December 1999

Category :	Commercial

General Area Served :	Sydney (NSW)

Service Licence Number :	SL4102

TECHNICAL SPECIFICATION - AM Radio

Specification Number :	TS4102001

Transmitter Site :-

Nominal location :	HOMEBUSH BAY

Australian Map Grid :	Zone	Easting	Northing

Reference	56	320998	6252302

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	MF-AM

Carrier Frequency :	954 kHz

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Elevation | Maximum CMF |
| --- | --- | --- |
| At all angles of azimuth | 0 | 955 V |

Special Conditions :-

This facility can operate with a CMF not to exceed 1350 V for all angle of azimuth, from one hour after local sunrise, to one hour before local sunset as defined at the transmitter site on the 15th day of each month (day-night switching).

Operation of this facility in accordance with Day/Night Switching can commence after October 2000.

Attachment 1.11

LICENCE AREA PLAN : Sydney – December 1999

Category :	Commercial

General Area Served :	Sydney (NSW)

Service Licence Number :	SL4100

TECHNICAL SPECIFICATION - AM Radio

Specification Number :	TS4100001

Transmitter Site :-

Nominal location :	HOMEBUSH

Australian Map Grid :	Zone	Easting	Northing

Reference	56	320600	6253899

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	MF-AM

Carrier Frequency :	1017 kHz

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Elevation | Maximum CMF |
| --- | --- | --- |
| At all angles of azimuth | 0 | 885 V |

Special Conditions :-

This facility can operate with a CMF not to exceed 1250 V for all angle of azimuth, from one hour after local sunrise, to one hour before local sunset as defined at the transmitter site on the 15th day of each month (day-night switching).

Operation of this facility in accordance with Day/Night Switching can commence after October 2000.

Attachment 1.12

LICENCE AREA PLAN : Sydney – December 1999

Category :	Commercial

General Area Served :	Sydney (NSW)

Service Licence Number :	SL4098

TECHNICAL SPECIFICATION - AM Radio

Specification Number :	TS4098001

Transmitter Site :-

Nominal location :	Salt Marsh HOMEBUSH BAY

Australian Map Grid :	Zone	Easting	Northing

Reference	56	322050	6253750

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	MF-AM

Carrier Frequency :	1170 kHz

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Elevation | Maximum CMF |
| --- | --- | --- |
| At all angles of azimuth | 0 | 920 V |

Special Conditions :-

This facility can operate with a CMF not to exceed 1300 V for all angle of azimuth, from one hour after local sunrise, to one hour before local sunset as defined at the transmitter site on the 15th day of each month (day-night switching).

Operation of this facility in accordance with Day/Night Switching can commence after October 2000.

Attachment 1.13

LICENCE AREA PLAN : Sydney – December 1999

Category :	Commercial

General Area Served :	Sydney (NSW)

Service Licence Number :	SL4101

TECHNICAL SPECIFICATION - AM Radio

Specification Number :	TS4101001

Transmitter Site :-

Nominal location :	Homebush Abbattoirs HOMEBUSH

Australian Map Grid :	Zone	Easting	Northing

Reference	56	321420	6254600

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	MF-AM

Carrier Frequency :	1269 kHz

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Elevation | Maximum CMF |
| --- | --- | --- |
| At all angles of azimuth | 0 | 955 V |

Special Conditions :-

This facility can operate with a CMF not to exceed 1350 V for all angle of azimuth, from one hour after local sunrise, to one hour before local sunset as defined at the transmitter site on the 15th day of each month (day-night switching).

Operation of this facility in accordance with Day/Night Switching can commence after October 2000.

Attachment 1.14

LICENCE AREA PLAN : Sydney – December 1999

Category :	Commercial

General Area Served :	Sydney (NSW)

Service Licence Number :	SL1150481

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10009024

Transmitter Site :-

Nominal location :	Ch10 Tower  ARTARMON

Australian Map Grid :	Zone	Easting	Northing

Reference	56	331500	6257730

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	95.3 MHz

Polarisation	Mixed

Maximum antenna height	170 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 15°T | 150 kW |
| 15°T - 20°T | 25 kW |
| 20°T - 200°T | 50 kW |
| 200°T - 205°T | 25 kW |
| 205°T - 275°T | 150 kW |
| 275°T - 275°T | 55 kW |
| 275°T - 360°T | 150 kW |

Special Conditions :-

The ABA will allocate this service four years after the date of publication of the Gazette notice that the Sydney radio licence area plan has been determined

Attachment 1.15

LICENCE AREA PLAN : Sydney – December 1999

Category :	Commercial

General Area Served :	Sydney (NSW)

Service Licence Number :	SL1150104

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS6229001

Transmitter Site :-

Nominal location :	Ch10 Tower  ARTARMON

Australian Map Grid :	Zone	Easting	Northing

Reference	56	331500	6257730

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	96.9 MHz

Polarisation	Mixed

Maximum antenna height	170 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 15°T | 150 kW |
| 15°T - 20°T | 25 kW |
| 20°T - 200°T | 50 kW |
| 200°T - 205°T | 25 kW |
| 205°T - 275°T | 150 kW |
| 275°T - 275°T | 55 kW |
| 275°T - 360°T | 150 kW |

Attachment 1.16

LICENCE AREA PLAN : Sydney – December 1999

Category :	Commercial

General Area Served :	Sydney (NSW)

Service Licence Number :	SL3032

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS3032004

Transmitter Site :-

Nominal location :	Ch10 Tower  ARTARMON

Australian Map Grid :	Zone	Easting	Northing

Reference	56	331500	6257730

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	104.1 MHz

Polarisation	Mixed

Maximum antenna height	170 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 15°T | 150 kW |
| 15°T - 20°T | 25 kW |
| 20°T - 200°T | 50 kW |
| 200°T - 205°T | 25 kW |
| 205°T - 275°T | 150 kW |
| 275°T - 275°T | 55 kW |
| 275°T - 360°T | 150 kW |

Attachment 1.17

LICENCE AREA PLAN : Sydney – December 1999

Category :	Commercial

General Area Served :	Sydney (NSW)

Service Licence Number :	SL3033

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS3033004

Transmitter Site :-

Nominal location :	Ch10 Tower  ARTARMON

Australian Map Grid :	Zone	Easting	Northing

Reference	56	331500	6257730

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	104.9 MHz

Polarisation	Mixed

Maximum antenna height	170 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 15°T | 150 kW |
| 15°T - 20°T | 25 kW |
| 20°T - 200°T | 50 kW |
| 200°T - 205°T | 25 kW |
| 205°T - 275°T | 150 kW |
| 275°T - 275°T | 55 kW |
| 275°T - 360°T | 150 kW |

Attachment 1.18

LICENCE AREA PLAN : Sydney – December 1999

Category :	Commercial

General Area Served :	Sydney (NSW)

Service Licence Number :	SL4103

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS4103001

Transmitter Site :-

Nominal location :	Ch10 Tower  ARTARMON

Australian Map Grid :	Zone	Easting	Northing

Reference	56	331500	6257730

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	106.5 MHz

Polarisation	Mixed

Maximum antenna height	170 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 15°T | 150 kW |
| 15°T - 20°T | 25 kW |
| 20°T - 200°T | 50 kW |
| 200°T - 205°T | 25 kW |
| 205°T - 275°T | 150 kW |
| 275°T - 275°T | 55 kW |
| 275°T - 360°T | 150 kW |

Attachment 1.19

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Sydney (NSW)

Service Licence Number :	SL5157

TECHNICAL SPECIFICATION - AM Radio

Specification Number :	TS5157002

Transmitter Site :-

Nominal location :	Church Lane PROSPECT

Australian Map Grid :	Zone	Easting	Northing

Reference	56	306900	6257000

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	MF-AM

Carrier Frequency :	1224 kHz

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Elevation | Maximum CMF |
| --- | --- | --- |
| 0°T - 0°T | 0 | 1.025 kV |
| 10°T - 10°T | 0 | 1.035 kV |
| 20°T - 20°T | 0 | 930 V |
| 30°T - 30°T | 0 | 700 V |
| 40°T - 40°T | 0 | 435 V |
| 50°T - 50°T | 0 | 455 V |
| 60°T - 60°T | 0 | 770 V |
| 70°T - 70°T | 0 | 1.06 kV |
| 80°T - 80°T | 0 | 1.225 kV |
| 90°T - 90°T | 0 | 1.26 kV |
| 100°T - 100°T | 0 | 1.195 kV |
| 110°T - 110°T | 0 | 1.08 kV |
| 120°T - 120°T | 0 | 955 V |
| 130°T - 130°T | 0 | 860 V |
| 140°T - 140°T | 0 | 810 V |
| 150°T - 150°T | 0 | 800 V |
| 160°T - 160°T | 0 | 840 V |
| 170°T - 170°T | 0 | 910 V |
| 180°T - 180°T | 0 | 980 V |
| 190°T - 190°T | 0 | 1.005 kV |
| 200°T - 200°T | 0 | 935 V |
| 210°T - 210°T | 0 | 735 V |
| 220°T - 220°T | 0 | 410 V |
| 230°T - 230°T | 0 | 65 V |
| 240°T - 240°T | 0 | 405 V |
| 250°T - 250°T | 0 | 725 V |
| 260°T - 260°T | 0 | 920 V |
| 270°T - 270°T | 0 | 980 V |
| 280°T - 280°T | 0 | 940 V |
| 290°T - 290°T | 0 | 850 V |
| 300°T - 300°T | 0 | 755 V |
| 310°T - 310°T | 0 | 695 V |
| 320°T - 320°T | 0 | 690 V |
| 330°T - 330°T | 0 | 740 V |
| 340°T - 340°T | 0 | 830 V |
| 350°T - 350°T | 0 | 940 V |

Attachment 1.20

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Sydney (NSW)

Service Licence Number :	SL1150110

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS4825001

Transmitter Site :-

Nominal location :	Ch10 Tower  ARTARMON

Australian Map Grid :	Zone	Easting	Northing

Reference	56	331500	6257730

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	92.1 MHz

Polarisation	Mixed

Maximum antenna height	160 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 15 kW |

Special Conditions :-

The service is not available until the clearance of 2MWM Manly South from 92.1 MHz to 90.3 MHz, described under attachments 10.4 and 10.5. The service is not available until the clearance of 2MWM Manly North from 93.7 MHz to 88.7 MHz, described under attachments 10.2 and 10.3. This service is also not available until the clearance of 2NSB Chatswood from 91.5 MHz to 99.3 MHz, described under attachments 7.2 and 7.3.

The service is required to protect the reception of NBN3 Newcastle and WIN3 Wollongong within their designated commercial television licence areas, only if no alternative television channel is available.

Attachment 1.21

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Sydney (NSW)

Service Licence Number :	SL1150111

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS6228001

Transmitter Site :-

Nominal location :	Ch10 Tower  ARTARMON

Australian Map Grid :	Zone	Easting	Northing

Reference	56	331500	6257730

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	93.7 MHz

Polarisation	Mixed

Maximum antenna height	160 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 15°T | 50 kW |
| 15°T - 20°T | 25 kW |
| 20°T - 200°T | 50 kW |
| 200°T - 205°T | 25 kW |
| 205°T - 360°T | 50 kW |

Special Conditions :-

The service is not available until the clearance of 2MWM Manly North from 93.7 MHz to 88.7 MHz, described under attachments 10.2 and 10.3. The service is not available until the clearance of 2MWM Manly South from 92.1 MHz to 90.3 MHz, described under attachments 10.4 and 10.5.  This service is also not available until the clearance of 2NSB Chatswood from 91.5 MHz to 99.3 MHz, described under attachments 7.2 and 7.3.

The service is required to protect the reception of 2SNO Goulburn with its designated commercial radio licence area.

Attachment 1.22

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Sydney (NSW)

Service Licence Number :	SL1150109

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS4826001

Transmitter Site :-

Nominal location :	Ch10 Tower  ARTARMON

Australian Map Grid :	Zone	Easting	Northing

Reference	56	331500	6257730

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	94.5 MHz

Polarisation	Mixed

Maximum antenna height	170 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 15°T | 150 kW |
| 15°T - 20°T | 25 kW |
| 20°T - 200°T | 50 kW |
| 200°T - 205°T | 25 kW |
| 205°T - 275°T | 150 kW |
| 275°T - 275°T | 55 kW |
| 275°T - 360°T | 150 kW |

Special Conditions :-

The service is not available until the clearance of 2MWM Manly North from 93.7 MHz to 88.7 MHz, described under attachments 10.2 and 10.3. The service is not available until the clearance of 2MWM Manly South from 92.1 MHz to 90.3 MHz, described under attachments 10.4 and 10.5.  This service is also not available until the clearance of 2NSB Chatswood from 91.5 MHz to 99.3 MHz, described under attachments 7.2 and 7.3.

Attachment 1.23

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Sydney (NSW)

Service Licence Number :	SL2290

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10002290

Transmitter Site :-

Nominal location :	Optus Tower Corner Miller Pacific Hwy NORTH 	SYDNEY

Australian Map Grid :	Zone	Easting	Northing

Reference	56	333990	6254000

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	98.5 MHz

Polarisation	Mixed

Maximum antenna height	140 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 15°T | 25 kW |
| 15°T - 20°T | 12.5 kW |
| 20°T - 200°T | 25 kW |
| 200°T - 205°T | 12.5 kW |
| 205°T - 360°T | 25 kW |

Attachment 1.24

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Sydney (NSW)

Service Licence Number :	SL3047

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10004307

Transmitter Site :-

Nominal location :	Governor Phillip Tower 1 Farrer Place  SYDNEY

Australian Map Grid :	Zone	Easting	Northing

Reference	56	334446	6251292

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	102.5 MHz

Polarisation	Mixed

Maximum antenna height	240 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 15°T | 50 kW |
| 15°T - 20°T | 25 kW |
| 20°T - 200°T | 50 kW |
| 200°T - 205°T | 25 kW |
| 205°T - 360°T | 50 kW |

Attachment 1.25

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Sydney (NSW)

Service Licence Number :	SL3044

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS3044001

Transmitter Site :-

Nominal location :	Miller St SYDNEY

Australian Map Grid :	Zone	Easting	Northing

Reference	56	334000	6254300

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	103.2 MHz

Polarisation	Mixed

Maximum antenna height	150 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 15°T | 50 kW |
| 15°T - 20°T | 25 kW |
| 20°T - 200°T | 50 kW |
| 200°T - 205°T | 25 kW |
| 205°T - 360°T | 50 kW |

Attachment 1.26

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Sydney (NSW)

Service Licence Number :	SL3059

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS3059001

Transmitter Site :-

Nominal location :	University of Technology  ULTIMO

Australian Map Grid :	Zone	Easting	Northing

Reference	56	333400	6249200

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	107.3 MHz

Polarisation	Mixed

Maximum antenna height	140 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 15°T | 50 kW |
| 15°T - 20°T | 25 kW |
| 20°T - 200°T | 50 kW |
| 200°T - 205°T | 25 kW |
| 205°T - 360°T | 50 kW |

Attachment 1.27

LICENCE AREA PLAN : Sydney – December 1999

Category :	Open Narrowcasting

General Area Served :	Sydney (NSW)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - AM Radio

Specification Number :	TS10008323

Transmitter Site :-

Nominal location :	HOMEBUSH

Australian Map Grid :	Zone	Easting	Northing

Reference	56	320600	6253899

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	MF-AM

Carrier Frequency :	1386 kHz

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Elevation | Maximum CMF |
| --- | --- | --- |
| 0°T - 0°T | 0 | 230 V |
| 10°T - 10°T | 0 | 85 V |
| 20°T - 20°T | 0 | 245 V |
| 30°T - 30°T | 0 | 480 V |
| 40°T - 40°T | 0 | 695 V |
| 50°T - 50°T | 0 | 870 V |
| 60°T - 60°T | 0 | 975 V |
| 70°T - 70°T | 0 | 1.015 kV |
| 80°T - 80°T | 0 | 990 V |
| 90°T - 90°T | 0 | 920 V |
| 100°T - 100°T | 0 | 830 V |
| 110°T - 110°T | 0 | 740 V |
| 120°T - 120°T | 0 | 670 V |
| 130°T - 130°T | 0 | 630 V |
| 140°T - 140°T | 0 | 625 V |
| 150°T - 150°T | 0 | 660 V |
| 160°T - 160°T | 0 | 725 V |
| 170°T - 170°T | 0 | 810 V |
| 180°T - 180°T | 0 | 900 V |
| 190°T - 190°T | 0 | 975 V |
| 200°T - 200°T | 0 | 1.015 kV |
| 210°T - 210°T | 0 | 990 V |
| 220°T - 220°T | 0 | 895 V |
| 230°T - 230°T | 0 | 735 V |
| 240°T - 240°T | 0 | 525 V |
| 250°T - 250°T | 0 | 290 V |
| 260°T - 260°T | 0 | 95 V |
| 270°T - 270°T | 0 | 195 V |
| 280°T - 280°T | 0 | 360 V |
| 290°T - 290°T | 0 | 490 V |
| 300°T - 300°T | 0 | 570 V |
| 310°T - 310°T | 0 | 615 V |
| 320°T - 320°T | 0 | 620 V |
| 330°T - 330°T | 0 | 585 V |
| 340°T - 340°T | 0 | 510 V |
| 350°T - 350°T | 0 | 390 V |

Special Conditions :-

The 'coverage radius' of this transmission is 50 kilometres measured from a point with the following AMG co-ordinates: Zone: 56; Easting: 320600; Northing: 6253899.  This point is the same as the nominal transmitter site.

Attachment 1.28

LICENCE AREA PLAN : Sydney – December 1999

Category :	Open Narrowcasting

General Area Served :	Penrith (NSW)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - AM Radio

Specification Number :	TS10005070

Transmitter Site :-

Nominal location :	5km NW of Penrith  EMU PLAINS

Australian Map Grid :	Zone	Easting	Northing

Reference	56	283200	6264500

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	MF-AM

Carrier Frequency :	1476 kHz

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Elevation | Maximum CMF |
| --- | --- | --- |
| At all angles of azimuth | 0 | 225 V |

Special Conditions :-

The 'coverage radius' of this transmission is 15 kilometres measured from a point with the following AMG co-ordinates: Zone: 56; Easting: 283200; Northing: 6264500.  This point is the same as the nominal transmitter site.

Attachment 1.29

LICENCE AREA PLAN : Sydney – December 1999

Category :	Open Narrowcasting

General Area Served :	Sydney (NSW)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - AM Radio

Specification Number :	TS8671001

Transmitter Site :-

Nominal location :	Bicentennial Park CONCORD WEST

Australian Map Grid :	Zone	Easting	Northing

Reference	56	322700	6252800

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	MF-AM

Carrier Frequency :	1539 kHz

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Elevation | Maximum CMF |
| --- | --- | --- |
| At all angles of azimuth | 0 | 385 V |

Special Conditions :-

The 'coverage radius' of this transmission is 20 kilometres measured from a point with the following AMG co-ordinates: Zone: 56; Easting: 283200; Northing: 6264500.  This point is the same as the nominal transmitter site.

Attachment 2.1

LICENCE AREA - WESTERN SUBURBS SYDNEY RA1

LICENCE AREA FOR SERVICE LICENCE NO :	SL4104

COMMERCIAL, WESTERN SUBURBS RA1

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

Blacktown (C)

Fairfield (C)

Penrith (C)

Campbelltown (C)

Parramatta (C)

Liverpool (C)

Baulkham Hills (A)

Holroyd (C)

Auburn (A)

Camden (A)

NSW CD 1201609

NSW CD 1231909

NSW CD 1230709

NSW CD 1230707

NSW CD 1231002

NSW CD 1230410

NSW CD 1231908

NSW CD 1230401

NSW CD 1201203

NSW CD 1201601

NSW CD 1231204

NSW CD 1201505

NSW CD 1231007

NSW CD 1230208

NSW CD 1201614

NSW CD 1201405

NSW CD 1231112

NSW CD 1230207

NSW CD 1201605

NSW CD 1231305

NSW CD 1201511

NSW CD 1230905

NSW CD 1231008

NSW CD 1231107

NSW CD 1231208

NSW CD 1231005

NSW CD 1201207

NSW CD 1231905

NSW CD 1231103

NSW CD 1231006

NSW CD 1201303

NSW CD 1230608

NSW CD 1230909

NSW CD 1230204

NSW CD 1201406

NSW CD 1230606

NSW CD 1201506

NSW CD 1231205

NSW CD 1231913

NSW CD 1201307

NSW CD 1231902

NSW CD 1230603

NSW CD 1230510

NSW CD 1231210

NSW CD 1201508

NSW CD 1230604

NSW CD 1230309

NSW CD 1201301

NSW CD 1232004

NSW CD 1231105

NSW CD 1230609

NSW CD 1230405

NSW CD 1230611

NSW CD 1201208

NSW CD 1231303

NSW CD 1231910

NSW CD 1230912

NSW CD 1231304

NSW CD 1230409

NSW CD 1201408

NSW CD 1201206

NSW CD 1201509

NSW CD 1230708

NSW CD 1230503

NSW CD 1230705

NSW CD 1201201

NSW CD 1230913

NSW CD 1231310

NSW CD 1230304

NSW CD 1231109

NSW CD 1201202

NSW CD 1230408

NSW CD 1231209

NSW CD 1231009

NSW CD 1231012

NSW CD 1230915

NSW CD 1231104

NSW CD 1231108

NSW CD 1230302

NSW CD 1231901

NSW CD 1201502

NSW CD 1201503

NSW CD 1201409

NSW CD 1231101

NSW CD 1231904

NSW CD 1230501

NSW CD 1231301

NSW CD 1230906

NSW CD 1231003

NSW CD 1232009

NSW CD 1201603

NSW CD 1201402

NSW CD 1230908

NSW CD 1230310

NSW CD 1232005

NSW CD 1232003

NSW CD 1231203

NSW CD 1201602

NSW CD 1201604

NSW CD 1230607

NSW CD 1201308

NSW CD 1232008

NSW CD 1231308

NSW CD 1231206

NSW CD 1231013

NSW CD 1231907

NSW CD 1201209

NSW CD 1230605

NSW CD 1230206

NSW CD 1231912

NSW CD 1230311

NSW CD 1201312

NSW CD 1230903

NSW CD 1231001

NSW CD 1231207

NSW CD 1231202

NSW CD 1231211

NSW CD 1230512

NSW CD 1230307

NSW CD 1201610

NSW CD 1231102

NSW CD 1201204

NSW CD 1230202

NSW CD 1231306

NSW CD 1230406

NSW CD 1230312

NSW CD 1231307

NSW CD 1230308

NSW CD 1230706

NSW CD 1230411

NSW CD 1201501

NSW CD 1230506

NSW CD 1230507

NSW CD 1230404

NSW CD 1231312

NSW CD 1230303

NSW CD 1232001

NSW CD 1230101

NSW CD 1230203

NSW CD 1201612

NSW CD 1230601

NSW CD 1230108

NSW CD 1230212

NSW CD 1231004

NSW CD 1201608

NSW CD 1231906

NSW CD 1231309

NSW CD 1230602

NSW CD 1230904

NSW CD 1201305

NSW CD 1231010

NSW CD 1231111

NSW CD 1231313

NSW CD 1230914

NSW CD 1232010

NSW CD 1230504

NSW CD 1230106

NSW CD 1230508

NSW CD 1201613

NSW CD 1230110

NSW CD 1231201

NSW CD 1230105

NSW CD 1230702

NSW CD 1201310

NSW CD 1230111

NSW CD 1410402

NSW CD 1231106

NSW CD 1230112

NSW CD 1230610

NSW CD 1230402

NSW CD 1230301

NSW CD 1230502

NSW CD 1231302

NSW CD 1201205

NSW CD 1201606

NSW CD 1230911

NSW CD 1201309

NSW CD 1201210

NSW CD 1230205

NSW CD 1201411

NSW CD 1230701

NSW CD 1230704

NSW CD 1230313

NSW CD 1201510

NSW CD 1230511

NSW CD 1231903

NSW CD 1230505

NSW CD 1231311

NSW CD 1232007

NSW CD 1230201

NSW CD 1230703

NSW CD 1201507

NSW CD 1230209

NSW CD 1230210

NSW CD 1230314

NSW CD 1201407

NSW CD 1231011

NSW CD 1230306

NSW CD 1231014

NSW CD 1230910

NSW CD 1230403

NSW CD 1230305

NSW CD 1201302

NSW CD 1232011

NSW CD 1230509

NSW CD 1201306

NSW CD 1201504

NSW CD 1231110

NSW CD 1230107

NSW CD 1201304

NSW CD 1201607

NSW CD 1201611

NSW CD 1201311

NSW CD 1232006

NSW CD 1230113

NSW CD 1230907

NSW CD 1232002

NSW CD 1201211

NSW CD 1230902

NSW CD 1230407

NSW CD 1230901

NSW CD 1201401

NSW CD 1231911

NSW CD 1230211

Note:

Standard terminology used by the Australian Bureau of Statistics

(C) 	=  	City

(CD) 	=  	Collection District 
(A)	=	NSW Local Government Area (excluding Cities)

Attachment 2.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	Commercial

General Area Served :	Sydney (NSW)

Service Licence Number :	SL4104

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS4104001

Transmitter Site :-

Nominal location :	Ch10 Tower  ARTARMON

Australian Map Grid :	Zone	Easting	Northing

Reference	56	331500	6257730

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	101.7 MHz

Polarisation	Mixed

Maximum antenna height	182 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 15°T | 150 kW |
| 15°T - 20°T | 25 kW |
| 20°T - 200°T | 50 kW |
| 200°T - 205°T | 25 kW |
| 205°T - 275°T | 150 kW |
| 275°T - 275°T | 55 kW |
| 275°T - 360°T | 150 kW |

Attachment 2.3

LICENCE AREA PLAN : Sydney – December 1999

Category :	Commercial

General Area Served :	Camden (NSW)

Service Licence Number :	SL4104

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10009796

Transmitter Site :-

Nominal location :	Broadcast Site Mt Prudhoe RAZORBACK

Australian Map Grid :	Zone	Easting	Northing

Reference	56	284623	6219812

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	88.3 MHz

Polarisation	Mixed

Maximum antenna height	15 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 100 W |

Special Conditions :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

Attachment 2.4

LICENCE AREA PLAN : Sydney – December 1999

Category :	Commercial

General Area Served :	Richmond (NSW)

Service Licence Number :	SL4104

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10009797

Transmitter Site :-

Nominal location :	Broadcasting Site WINMARLEE

Australian Map Grid :	Zone	Easting	Northing

Reference	56	281592	6272608

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	99.1 MHz

Polarisation	Mixed

Maximum antenna height	15 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 100 W |

Special Conditions :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

Attachment 3.1

LICENCE AREA - CAMPBELLTOWN RA1

LICENCE AREA FOR SERVICE LICENCE NOS :	SL1150490, SL1150150

COMMERCIAL AND COMMUNITY, CAMPBELLTOWN RA1

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

Campbelltown (C)

Camden (A)

NSW CD 1201609

NSW CD 1201601

NSW CD 1201505

NSW CD 1201614

NSW CD 1201605

NSW CD 1201511

NSW CD 1201303

NSW CD 1201406

NSW CD 1201506

NSW CD 1201307

NSW CD 1201508

NSW CD 1201301

NSW CD 1201408

NSW CD 1201206

NSW CD 1201509

NSW CD 1201502

NSW CD 1201503

NSW CD 1201603

NSW CD 1201402

NSW CD 1201602

NSW CD 1201604

NSW CD 1201308

NSW CD 1201312

NSW CD 1201610

NSW CD 1201501

NSW CD 1201612

NSW CD 1201608

NSW CD 1201305

NSW CD 1201613

NSW CD 1201310

NSW CD 1201606

NSW CD 1201309

NSW CD 1201210

NSW CD 1201411

NSW CD 1201510

NSW CD 1201507

NSW CD 1201302

NSW CD 1201306

NSW CD 1201504

NSW CD 1201304

NSW CD 1201607

NSW CD 1201611

NSW CD 1201311

NSW CD 1201211

NSW CD 1201401

Note:

Standard terminology used by the Australian Bureau of Statistics

(C) 	=  	City

(CD) 	=  	Collection District

(A)	=	NSW Local Government Area (excluding Cities)

Attachment 3.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	Commercial

General Area Served :	Campbelltown (NSW)

Service Licence Number :	SL1150490

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10009041

Transmitter Site :-

Nominal location :	Lot 22 Mount Hercules Road Razorback Range via 	PICTON

Australian Map Grid :	Zone	Easting	Northing

Reference	56	285340	6218780

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	91.3 MHz

Polarisation	Vertical

Maximum antenna height	60 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 1 kW |

Special Conditions :-

This service is restricted to 200 W maximum over the sector 0°T – 90°T until the clearance of the Chatswood community radio service from 91.5 MHz to 99.3 MHz, described under attachments 7.2 and 7.3.

The service is required to protect the reception of NBN3 Newcastle and WIN3 Wollongong within their designated commercial television licence areas, only if no alternative television channel is available.

Attachment 3.3

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Campbelltown (NSW)

Service Licence Number :	SL1150150

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10008284

Transmitter Site :-

Nominal location :	Lot 22 Mount Hercules Road Razorback Range via 	PICTON

Australian Map Grid :	Zone	Easting	Northing

Reference	56	285340	6218780

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	100.3 MHz

Polarisation	Mixed

Maximum antenna height	30 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 200 W |

Special Conditions :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

The service is not available until the clearance of 2MWM Manly North from 93.7 MHz to 88.7 MHz, described under attachments 10.2 and 10.3. The service is not available until the clearance of 2MWM Manly South from 92.1 MHz to 90.3 MHz, described under attachments 10.4 and 10.5.  This service is also not available until the clearance of 2NSB Chatswood from 91.5 MHz to 99.3 MHz, described under attachments 7.2 and 7.3.

Attachment 4.1

LICENCE AREA - BANKSTOWN RA1

LICENCE AREA FOR SERVICE LICENCE NO:	SL3043

COMMUNITY, BANKSTOWN RA1

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

Bankstown (C)

Auburn (A)

NSW CD 1410402

Note:

Standard terminology used by the Australian Bureau of Statistics

(C) 	=  	City

(CD) 	=  	Collection District

(A)	=	NSW Local Government Area (excluding Cities)

Attachment 4.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Bankstown (NSW)

Service Licence Number :	SL3043

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS3043001

Transmitter Site :-

Nominal location :	Bankstown City Radio site Simmat Ave

CONDELL PARK

Australian Map Grid :	Zone	Easting	Northing

Reference	56	316090	6244800

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	88.7 MHz

Polarisation	Vertical

Maximum antenna height	20 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 360°T | 160 W |
| 90°T - 90°T | 50 W |
| 270°T - 270°T | 50 W |

Special Condition :-

This specification will have its frequency replaced by 100.9 MHz, described under attachment 4.3, at a date not later than six months after publication of the Gazette notice that the licence area plan for Sydney has been determined.

Attachment 4.3

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Bankstown (NSW)

Service Licence Number :	SL3043

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10009148

Transmitter Site :-

Nominal location :	Bankstown City Radio site Simmat Ave CONDELL 	PARK

Australian Map Grid :	Zone	Easting	Northing

Reference	56	316090	6244800

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	100.9 MHz

Polarisation	Mixed

Maximum antenna height	30 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 360°T | 200 W |
| 90°T - 90°T | 100 W |
| 270°T - 270°T | 100 W |

Special Condition :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

This specification will replace the frequency 88.7 MHz, described under attachment 4.2, at a date not later than six months after publication of the Gazette notice that the licence area plan for Sydney has been determined.

Attachment 5.1

LICENCE AREA - BLACKTOWN RA1

LICENCE AREA FOR SERVICE LICENCE NO :	SL1150152

COMMUNITY, BLACKTOWN RA1

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

Blacktown (C)

Fairfield (C)

Holroyd (C)

Note:

Standard terminology used by the Australian Bureau of Statistics

(C) 	=  	City

Attachment 5.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Blacktown (NSW)

Service Licence Number :	SL1150152

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10008287

Transmitter Site :-

Nominal location :	Waterboard Tower  Villiers Road  HORSLEY PARK

Australian Map Grid :	Zone	Easting	Northing

Reference	56	301070	6250870

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	99.9 MHz

Polarisation	Mixed

Maximum antenna height	30 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 135°T | 200 W |
| 135°T - 315°T | 50 W |
| 315°T - 360°T | 200 W |

Special Conditions :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

The service is not available until the clearance of 2MWM Manly North from 93.7 MHz to 88.7 MHz, described under attachments 10.2 and 10.3. The service is not available until the clearance of 2MWM Manly South from 92.1 MHz to 90.3 MHz, described under attachments 10.4 and 10.5.  This service is also not available until the clearance of 2NSB Chatswood from 91.5 MHz to 99.3 MHz, described under attachments 7.2 and 7.3.

Attachment 6.1

LICENCE AREA - BURWOOD RA1

LICENCE AREA FOR SERVICE LICENCE NO :	SL3054

COMMUNITY, BURWOOD RA1

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

Ashfield (A)

Drummoyne (A)

Burwood (A)

Concord (A)

NSW CD 1410506

NSW CD 1410606

NSW CD 1410501

NSW CD 1410605

NSW CD 1410503

NSW CD 1410707

NSW CD 1410411

NSW CD 1410609

NSW CD 1410610

NSW CD 1410704

NSW CD 1410502

NSW CD 1410412

NSW CD 1410504

NSW CD 1410410

NSW CD 1410710

NSW CD 1410405

NSW CD 1410406

NSW CD 1410701

NSW CD 1410601

NSW CD 1410407

NSW CD 1410604

NSW CD 1410703

NSW CD 1410702

NSW CD 1410409

NSW CD 1410709

NSW CD 1410602

NSW CD 1410408

NSW CD 1410505

NSW CD 1410607

NSW CD 1410608

NSW CD 1410507

NSW CD 1410508

NSW CD 1410511

NSW CD 1410711

NSW CD 1410705

NSW CD 1410603

NSW CD 1410404

NSW CD 1410706

NSW CD 1410708

NSW CD 1410401

NSW CD 1410509

NSW CD 1410403

NSW CD 1410510

Note:

Standard terminology used by the Australian Bureau of Statistics

- =	Collection District

(A)	=	NSW Local Government Area (excluding Cities)

Attachment 6.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Burwood (NSW)

Service Licence Number :	SL3054

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS3054001

Transmitter Site :-

Nominal location :	Church St BURWOOD

Australian Map Grid :	Zone	Easting	Northing

Reference	56	324600	6249200

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	88.1 MHz

Polarisation	Mixed

Maximum antenna height	30 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 360°T | 100 W |
| 220°T - 220°T | 12 W |

Special Conditions :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

Attachment 7.1

LICENCE AREA - CHATSWOOD RA1

LICENCE AREA FOR SERVICE LICENCE NO:	SL3052

COMMUNITY, CHATSWOOD RA1

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

Ku-ring-gai (A)

North Sydney (A)

Willoughby (C)

Lane Cove (A)

Mosman (A)

Note:

Standard terminology used by the Australian Bureau of Statistics

(C) 	=  	City

(A)	=	NSW Local Government Area (excluding Cities)

Attachment 7.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Chatswood (NSW)

Service Licence Number :	SL3052

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS3052001

Transmitter Site :-

Nominal location :	Victor Street CHATSWOOD

Australian Map Grid :	Zone	Easting	Northing

Reference	56	331625	6258630

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	91.5 MHz

Polarisation	Mixed

Maximum antenna height	95 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 45°T | 130 W |
| 45°T - 45°T | 40 W |
| 45°T - 225°T | 63 W |
| 225°T - 225°T | 40 W |
| 225°T - 360°T | 130 W |

Special Condition :-

This specification will have its frequency replaced by 99.3 MHz, described under attachment 7.3, at a date not later than one year after publication of the Gazette notice that the licence area plan for Sydney has been determined.

Attachment 7.3

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Chatswood (NSW)

Service Licence Number :	SL3052

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10008273

Transmitter Site :-

Nominal location :	Victor Street CHATSWOOD

Australian Map Grid :	Zone	Easting	Northing

Reference	56	331625	6258630

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	99.3 MHz

Polarisation	Mixed

Maximum antenna height	95 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 45°T | 200 W |
| 45°T - 60°T | 80 W |
| 60°T - 225°T | 200 W |
| 225°T - 240°T | 80 W |
| 240°T - 360°T | 200 W |

Special Condition :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

This specification will replace the frequency 91.5 MHz, described under attachment 7.2, at a date not later than one year after publication of the Gazette notice that the licence area plan for Sydney has been determined.

Attachment 8.1

LICENCE AREA - HORNSBY RA1

LICENCE AREA FOR SERVICE LICENCE NO :	SL1150154

COMMUNITY, HORNSBY RA1

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

Hornsby (A)

Ku-ring-gai (A)

Note:

Standard terminology used by the Australian Bureau of Statistics

(A)	NSW Local Government Area (excluding Cities)

Attachment 8.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Hornsby (NSW)

Service Licence Number :	SL1150154

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10008289

Transmitter Site :-

Nominal location :	10 Romsey St WAITARA

Australian Map Grid :	Zone	Easting	Northing

Reference	56	324061	6268196

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	100.1 MHz

Polarisation	Mixed

Maximum antenna height	30 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 90°T | 200 W |
| 90°T - 270°T | 50 W |
| 270°T - 360°T | 200 W |

Special Conditions :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

The service is not available until the clearance of 2MWM Manly North from 93.7 MHz to 88.7 MHz, described under attachments 10.2 and 10.3. The service is not available until the clearance of 2MWM Manly South from 92.1 MHz to 90.3 MHz, described under attachments 10.4 and 10.5.  This service is also not available until the clearance of 2NSB Chatswood from 91.5 MHz to 99.3 MHz, described under attachments 7.2 and 7.3.

Attachment 9.1

LICENCE AREA - LIVERPOOL RA1

LICENCE AREA FOR SERVICE LICENCE NO :	SL3046

COMMUNITY, LIVERPOOL RA1

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

Fairfield (C)

Liverpool (C)

Note:

Standard terminology used by the Australian Bureau of Statistics

(C)   =  City

Attachment 9.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Liverpool (NSW)

Service Licence Number :	SL3046

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS3046001

Transmitter Site :-

Nominal location :	MOUNT PRITCHARD

Australian Map Grid :	Zone	Easting	Northing

Reference	56	305700	6245799

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	89.3 MHz

Polarisation	Mixed

Maximum antenna height	30 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 80 W |

Attachment 10.1

LICENCE AREA - MANLY RA1

LICENCE AREA FOR SERVICE LICENCE NO :	SL3049

COMMUNITY, MANLY RA1

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

Warringah (A)

Pittwater (A)

Manly (A)

Note:

Standard terminology used by the Australian Bureau of Statistics

(A)	NSW Local Government Area (excluding Cities)

Attachment 10.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Manly North (NSW)

Service Licence Number :	SL3049

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS3049001

Transmitter Site :-

Nominal location :	Bilgola Plateau  Plateau Road

Australian Map Grid :	Zone	Easting	Northing

Reference	56	343500	6275804

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	93.7 MHz

Polarisation	Mixed

Maximum antenna height	20 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 12°T - 12°T | 20 W |
| 192°T - 192°T | 160 W |

Special Condition :-

This specification will have its frequency replaced by 88.7 MHz, described under attachment 10.3, at a date not later than one year after publication of the Gazette notice that the licence area plan for Sydney has been determined.

Attachment 10.3

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Manly North (NSW)

Service Licence Number :	SL3049

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10008216

Transmitter Site :-

Nominal location :	Bilgola Plateau  Plateau Road

Australian Map Grid :	Zone	Easting	Northing

Reference	56	343500	6275804

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	88.7 MHz

Polarisation	Mixed

Maximum antenna height	30 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T – 360°T | 200 W |
| 10°T – 10°T | 20 W |

Special Condition :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

This service cannot commence transmission until the clearance of the Bankstown community radio service from 88.7 MHz to 100.9 MHz, described under attachments 4.2 and 4.3.

This specification will replace the frequency 93.7 MHz, described under attachment 10.2, at a date not later than one year after publication of the Gazette that the licence area plan for Sydney has been determined.

Attachment 10.4

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Manly South (NSW)

Service Licence Number :	SL3049

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS3049002

Transmitter Site :-

Nominal location :	Tania Park BALGOWLAH HEIGHTS

Australian Map Grid :	Zone	Easting	Northing

Reference	56	339400	6257500

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	92.1 MHz

Polarisation	Mixed

Maximum antenna height	20 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 170°T - 170°T | 4 W |
| 350°T - 350°T | 40 W |

Special Condition :-

This specification will have its frequency replaced by 90.3 MHz, described under attachment 10.5, at a date not later than one year after publication of the Gazette that the licence area plan for Sydney has been determined.

Attachment 10.5

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Manly South (NSW)

Service Licence Number :	SL3049

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10008215

Transmitter Site :-

Nominal location :	Tania Park BALGOWLAH HEIGHTS

Australian Map Grid :	Zone	Easting	Northing

Reference	56	339400	6257500

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	90.3 MHz

Polarisation	Mixed

Maximum antenna height	30 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 360°T | 50 W |
| 170°T - 170°T | 5 W |

Special Condition :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

This specification will replace the frequency 92.1 MHz, described under attachment 10.4, at a date not later than one year after publication of the Gazette that the licence area plan for Sydney has been determined.

Attachment 11.1

LICENCE AREA - NARWEE RA1

LICENCE AREA FOR SERVICE LICENCE NO :	SL3050

COMMUNITY, NARWEE RA1

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

Canterbury (C)

Rockdale (C)

Marrickville (A)

Hurstville (C)

Kogarah (A)

Note:

Standard terminology used by the Australian Bureau of Statistics

(C)   =  City

(A)	= NSW Local Government Area (excluding Cities)

Attachment 11.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Narwee (NSW)

Service Licence Number :	SL3050

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS3050001

Transmitter Site :-

Nominal location :	Penshurst  Water Tower

Australian Map Grid :	Zone	Easting	Northing

Reference	56	323511	6239580

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	90.1 MHz

Polarisation	Mixed

Maximum antenna height	40 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T – 220°T | 200 W |
| 220°T – 230°T | 50 W |
| 230°T - 360°T | 200 W |

Special Conditions :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

Attachment 12.1

LICENCE AREA - PARRAMATTA RA1

LICENCE AREA FOR SERVICE LICENCE NO :	SL1251

COMMUNITY, PARRAMATTA RA1

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

Parramatta (C)

Holroyd (C)

NSW CD 1260508

NSW CD 1261009

NSW CD 1261601

NSW CD 1260505

NSW CD 1261405

NSW CD 1260601

NSW CD 1260506

NSW CD 1260504

NSW CD 1261605

NSW CD 1260310

NSW CD 1260603

NSW CD 1260403

NSW CD 1261305

NSW CD 1261002

NSW CD 1261203

NSW CD 1261208

NSW CD 1260409

NSW CD 1260710

NSW CD 1261402

NSW CD 1260701

NSW CD 1261207

NSW CD 1260808

NSW CD 1260507

NSW CD 1260503

NSW CD 1260304

NSW CD 1260606

NSW CD 1260708

NSW CD 1261401

NSW CD 1261409

NSW CD 1260404

NSW CD 1260607

NSW CD 1261308

NSW CD 1261606

NSW CD 1261102

NSW CD 1261413

NSW CD 1260705

NSW CD 1260802

NSW CD 1260903

NSW CD 1261209

NSW CD 1261505

NSW CD 1260904

NSW CD 1260602

NSW CD 1261406

NSW CD 1261607

NSW CD 1261303

NSW CD 1260405

NSW CD 1260410

NSW CD 1260709

NSW CD 1260901

NSW CD 1261204

NSW CD 1261210

NSW CD 1261006

NSW CD 1260311

NSW CD 1261007

NSW CD 1261506

NSW CD 1261507

NSW CD 1261502

NSW CD 1261106

NSW CD 1261508

NSW CD 1260402

NSW CD 1261403

NSW CD 1260509

NSW CD 1260408

NSW CD 1260511

NSW CD 1260605

NSW CD 1260706

NSW CD 1260609

NSW CD 1260804

NSW CD 1260809

NSW CD 1261109

NSW CD 1260703

NSW CD 1261503

NSW CD 1261211

NSW CD 1261501

NSW CD 1260704

NSW CD 1261202

NSW CD 1260806

NSW CD 1261306

NSW CD 1261509

NSW CD 1261008

NSW CD 1260805

NSW CD 1261206

NSW CD 1261602

NSW CD 1261608

NSW CD 1260309

NSW CD 1261309

NSW CD 1260604

NSW CD 1260811

NSW CD 1260807

NSW CD 1261304

NSW CD 1261104

NSW CD 1260406

NSW CD 1260810

NSW CD 1260502

NSW CD 1261108

NSW CD 1260501

NSW CD 1261301

NSW CD 1261105

NSW CD 1260302

NSW CD 1261310

NSW CD 1260707

NSW CD 1261407

NSW CD 1260906

NSW CD 1260203

NSW CD 1260803

NSW CD 1261010

NSW CD 1261603

NSW CD 1261412

NSW CD 1261302

NSW CD 1261201

NSW CD 1260307

NSW CD 1261103

NSW CD 1261205

NSW CD 1261001

NSW CD 1260907

NSW CD 1260908

NSW CD 1260611

NSW CD 1261410

NSW CD 1260106

NSW CD 1261504

NSW CD 1261609

NSW CD 1260902

NSW CD 1261003

NSW CD 1261107

NSW CD 1260407

NSW CD 1261610

NSW CD 1261705

NSW CD 1260401

NSW CD 1260610

NSW CD 1261510

NSW CD 1260204

NSW CD 1260812

NSW CD 1261307

NSW CD 1261611

NSW CD 1260801

NSW CD 1261709

NSW CD 1260702

NSW CD 1260209

NSW CD 1261005

NSW CD 1261706

NSW CD 1260107

NSW CD 1260510

NSW CD 1261707

NSW CD 1260909

NSW CD 1260905

NSW CD 1261110

NSW CD 1260910

NSW CD 1260512

NSW CD 1261404

NSW CD 1261604

NSW CD 1260210

NSW CD 1261212

NSW CD 1261708

NSW CD 1260207

NSW CD 1260306

NSW CD 1261703

NSW CD 1260308

NSW CD 1261004

NSW CD 1261704

NSW CD 1260206

NSW CD 1261701

NSW CD 1261411

NSW CD 1260301

NSW CD 1261702

NSW CD 1260312

NSW CD 1261408

NSW CD 1261414

NSW CD 1260608

NSW CD 1260313

NSW CD 1260205

NSW CD 1260305

NSW CD 1260303

NSW CD 1261101

Note:

Standard terminology used by the Australian Bureau of Statistics

(C)   =  City

(CD)  =  Collection District

Attachment 12.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Parramatta (NSW)

Service Licence Number :	SL1251

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS1251001

Transmitter Site :-

Nominal location :	Seven Hills Rd BAULKHAM HILLS

Australian Map Grid :	Zone	Easting	Northing

Reference	56	312900	6261800

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	90.5 MHz

Polarisation	Mixed

Maximum antenna height	40 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 90°T | 200 W |
| 90°T - 270°T | 60 W |
| 270°T - 360°T | 200 W |

Special Conditions :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

Attachment 13.1

LICENCE AREA - PENRITH RA1

LICENCE AREA FOR SERVICE LICENCE NO :	SL1150151

COMMUNITY, PENRITH RA1

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

Penrith (C)

NSW CD 1231002

NSW CD 1231204

NSW CD 1231007

NSW CD 1231112

NSW CD 1231305

NSW CD 1231008

NSW CD 1231107

NSW CD 1231208

NSW CD 1231005

NSW CD 1231103

NSW CD 1231006

NSW CD 1231205

NSW CD 1231210

NSW CD 1231105

NSW CD 1231303

NSW CD 1231304

NSW CD 1231310

NSW CD 1231109

NSW CD 1231209

NSW CD 1231009

NSW CD 1231012

NSW CD 1230915

NSW CD 1231104

NSW CD 1231108

NSW CD 1231101

NSW CD 1231301

NSW CD 1230906

NSW CD 1231003

NSW CD 1231203

NSW CD 1231308

NSW CD 1231206

NSW CD 1231013

NSW CD 1231001

NSW CD 1231207

NSW CD 1231202

NSW CD 1231211

NSW CD 1231306

NSW CD 1231307

NSW CD 1231312

NSW CD 1231004

NSW CD 1231309

NSW CD 1231010

NSW CD 1231111

NSW CD 1231313

NSW CD 1231201

NSW CD 1231106

NSW CD 1231302

NSW CD 1231311

NSW CD 1231011

NSW CD 1231014

NSW CD 1231110

Note:

Standard terminology used by the Australian Bureau of Statistics

(C)   =  City

(CD)  =  Collection District

Attachment 13.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Penrith (NSW)

Service Licence Number :	SL1150151

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10008286

Transmitter Site :-

Nominal location :	University of Western Sydney Nepean  WERRINGTON

Australian Map Grid :	Zone	Easting	Northing

Reference	56	290660	6261350

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	100.7 MHz

Polarisation	Mixed

Maximum antenna height	30 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 200 W |

Special Conditions :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

The service is not available until the clearance of 2MWM Manly North from 93.7 MHz to 88.7 MHz, described under attachments 10.2 and 10.3. The service is not available until the clearance of 2MWM Manly South from 92.1 MHz to 90.3 MHz, described under attachments 10.4 and 10.5.  This service is also not available until the clearance of 2NSB Chatswood from 91.5 MHz to 99.3 MHz, described under attachments 7.2 and 7.3.

Attachment 14.1

LICENCE AREA - RYDE RA1

LICENCE AREA FOR SERVICE LICENCE NO :	SL3057

COMMUNITY, RYDE RA1

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

Ryde (C)

Hunters Hill (A)

Note:

Standard terminology used by the Australian Bureau of Statistics

(C)   =  City

(A)   =  NSW Local Government Area (excluding Cities)

Attachment 14.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Ryde (NSW)

Service Licence Number :	SL3057

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS3057001

Transmitter Site :-

Nominal location :	Goulding Road Trig Point NORTH RYDE

Australian Map Grid :	Zone	Easting	Northing

Reference	56	325440	6258130

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	88.5 MHz

Polarisation	Mixed

Maximum antenna height	30 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 100 W |

Special Conditions :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

Attachment 15.1

LICENCE AREA - SUTHERLAND RA1

LICENCE AREA FOR SERVICE LICENCE NO :	SL4842

COMMUNITY, SUTHERLAND RA1

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

Sutherland Shire (A)

Note:

Standard terminology used by the Australian Bureau of Statistics

(A) = NSW Local Government Area (excluding Cities)

Attachment 15.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Sutherland (NSW)

Service Licence Number :	SL4842

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS4842001

Transmitter Site :-

Nominal location :	Eton St SUTHERLAND

Australian Map Grid :	Zone	Easting	Northing

Reference	56	320800	6232500

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	99.7 MHz

Polarisation	Mixed

Maximum antenna height	30 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 320°T | 200 W |
| 320°T - 350°T | 50 W |
| 350°T - 360°T | 200 W |

Special Conditions :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

Attachment 16.1

LICENCE AREA - SYDNEY RA2

LICENCE AREA FOR SERVICE LICENCE NOS :	SL3058

COMMUNITY, SYDNEY RA2

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

South Sydney (C)

Marrickville (A)

Leichhardt (A)

Sydney (C)

NSW CD 1421502

NSW CD 1421508

NSW CD 1421908

NSW CD 1421603

NSW CD 1421702

NSW CD 1421902

NSW CD 1421803

NSW CD 1421506

NSW CD 1421909

NSW CD 1421707

NSW CD 1421804

NSW CD 1421704

NSW CD 1421701

NSW CD 1421710

NSW CD 1421811

NSW CD 1421612

NSW CD 1421906

NSW CD 1421806

NSW CD 1421807

NSW CD 1421706

NSW CD 1421709

NSW CD 1421511

NSW CD 1421609

NSW CD 1421613

NSW CD 1421907

NSW CD 1421805

NSW CD 1421708

NSW CD 1421607

NSW CD 1421809

NSW CD 1421910

NSW CD 1421610

NSW CD 1421505

NSW CD 1421813

NSW CD 1421912

NSW CD 1421602

NSW CD 1421507

NSW CD 1421510

NSW CD 1421604

NSW CD 1421911

NSW CD 1421608

NSW CD 1421605

NSW CD 1421512

NSW CD 1421810

NSW CD 1421711

NSW CD 1421705

NSW CD 1421808

NSW CD 1421802

NSW CD 1421903

NSW CD 1421503

NSW CD 1421801

NSW CD 1421509

NSW CD 1421703

NSW CD 1421601

NSW CD 1421611

NSW CD 1421812

NSW CD 1421606

NSW CD 1421901

NSW CD 1421905

NSW CD 1421614

NSW CD 1421504

NSW CD 1421501

Note:

Standard terminology used by the Australian Bureau of Statistics

(C)   =  City

(A))  =  NSW Local Government Area (excluding Cities)

Attachment 16.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Sydney (NSW)

Service Licence Number :	SL3058

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS3058001

Transmitter Site :-

Nominal location :	Sydney Town Hall SYDNEY

Australian Map Grid :	Zone	Easting	Northing

Reference	56	334000	6250300

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	88.9 MHz

Polarisation	Mixed

Maximum antenna height	130 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 360°T | 200 W |
| 45°T - 45°T | 20 W |

Special Condition :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

The service is restricted to a maximum ERP of 70 W, until the clearance of 2BCR Bankstown from 88.7 MHz to 100.5 MHz, described under attachments 4.2 and 4.3.

The service is restricted to a maximum ERP of 10 W at the sector 45°T, until the clearance of 2MWM Manly North from 93.7 MHz to 88.7 MHz, described under attachments 10.2 and 10.3.

Attachment 17.1

LICENCE AREA - WAVERLEY RA1

LICENCE AREA FOR SERVICE LICENCE NO :	SL3056

COMMUNITY, WAVERLEY RA1

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

Randwick (C)

South Sydney (C)

Waverley (A)

Woollahra (A)

Sydney (C)

NSW CD 1421502

NSW CD 1421508

NSW CD 1421908

NSW CD 1421603

NSW CD 1421702

NSW CD 1421902

NSW CD 1421803

NSW CD 1421506

NSW CD 1421909

NSW CD 1421707

NSW CD 1421804

NSW CD 1421704

NSW CD 1421701

NSW CD 1421710

NSW CD 1421811

NSW CD 1421612

NSW CD 1421906

NSW CD 1421806

NSW CD 1421807

NSW CD 1421706

NSW CD 1421709

NSW CD 1421511

NSW CD 1421609

NSW CD 1421613

NSW CD 1421907

NSW CD 1421805

NSW CD 1421708

NSW CD 1421607

NSW CD 1421809

NSW CD 1421910

NSW CD 1421610

NSW CD 1421505

NSW CD 1421813

NSW CD 1421912

NSW CD 1421602

NSW CD 1421507

NSW CD 1421510

NSW CD 1421604

NSW CD 1421911

NSW CD 1421608

NSW CD 1421605

NSW CD 1421512

NSW CD 1421810

NSW CD 1421711

NSW CD 1421705

NSW CD 1421808

NSW CD 1421802

NSW CD 1421903

NSW CD 1421503

NSW CD 1421801

NSW CD 1421509

NSW CD 1421703

NSW CD 1421601

NSW CD 1421611

NSW CD 1421812

NSW CD 1421606

NSW CD 1421901

NSW CD 1421905

NSW CD 1421614

NSW CD 1421504

NSW CD 1421501

Note:

Standard terminology used by the Australian Bureau of Statistics

(C)   =  City

- =  Collection District

(A)	=  NSW Local Government Area (excluding Cities)

Attachment 17.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Waverley (NSW)

Service Licence Number :	SL3056

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS3056001

Transmitter Site :-

Nominal location :	Waverley College Birrell WAVERLEY

Australian Map Grid :	Zone	Easting	Northing

Reference	56	338600	6247700

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	89.7 MHz

Polarisation	Mixed

Maximum antenna height	30 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 200 W |

Special Condition :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

This specification is limited to a maximum ERP of 160 W over the sector 270°T - 360°T until the clearance of the 2VTR Windsor community radio service from 89.7 MHz to 89.9 MHz, refer to attachments 18.2 and 18.3.

Attachment 18.1

LICENCE AREA - WINDSOR RA1

LICENCE AREA FOR SERVICE LICENCE NO :	SL5943

COMMUNITY, WINDSOR RA1

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 6 August 1996, is:

Area Description

NSW CD 1231909

NSW CD 1230709

NSW CD 1230707

NSW CD 1230410

NSW CD 1231908

NSW CD 1230401

NSW CD 1230208

NSW CD 1230207

NSW CD 1231905

NSW CD 1230608

NSW CD 1230204

NSW CD 1230606

NSW CD 1231913

NSW CD 1231902

NSW CD 1230603

NSW CD 1230510

NSW CD 1230604

NSW CD 1230309

NSW CD 1232004

NSW CD 1230609

NSW CD 1230405

NSW CD 1230611

NSW CD 1231910

NSW CD 1230409

NSW CD 1230708

NSW CD 1230503

NSW CD 1230705

NSW CD 1230304

NSW CD 1230408

NSW CD 1230302

NSW CD 1231901

NSW CD 1231904

NSW CD 1230501

NSW CD 1232009

NSW CD 1230310

NSW CD 1232005

NSW CD 1232003

NSW CD 1230607

NSW CD 1232008

NSW CD 1231907

NSW CD 1230605

NSW CD 1230206

NSW CD 1231912

NSW CD 1230311

NSW CD 1230512

NSW CD 1230307

NSW CD 1230202

NSW CD 1230406

NSW CD 1230312

NSW CD 1230308

NSW CD 1230706

NSW CD 1230411

NSW CD 1230506

NSW CD 1230507

NSW CD 1230404

NSW CD 1230303

NSW CD 1232001

NSW CD 1230101

NSW CD 1230203

NSW CD 1230601

NSW CD 1230108

NSW CD 1230212

NSW CD 1231906

NSW CD 1230602

NSW CD 1232010

NSW CD 1230504

NSW CD 1230106

NSW CD 1230508

NSW CD 1230110

NSW CD 1230105

NSW CD 1230702

NSW CD 1230111

NSW CD 1230112

NSW CD 1230610

NSW CD 1230402

NSW CD 1230301

NSW CD 1230502

NSW CD 1230109

NSW CD 1230205

NSW CD 1230701

NSW CD 1230704

NSW CD 1230313

NSW CD 1230511

NSW CD 1231903

NSW CD 1230505

NSW CD 1232007

NSW CD 1230201

NSW CD 1230703

NSW CD 1230209

NSW CD 1230210

NSW CD 1230314

NSW CD 1230306

NSW CD 1230403

NSW CD 1230305

NSW CD 1232011

NSW CD 1230509

NSW CD 1230107

NSW CD 1232006

NSW CD 1230113

NSW CD 1232002

NSW CD 1230407

NSW CD 1230104

NSW CD 1231911

NSW CD 1230211

Note:

Standard terminology used by the Australian Bureau of Statistics

(CD)  =  Collection District

Attachment 18.2

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Windsor (NSW)

Service Licence Number :	SL5943

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10002532

Transmitter Site :-

Nominal location :	Miles Comms Old Site 246 Burralow Road 	KURRAJONG HEIGHTS

Australian Map Grid :	Zone	Easting	Northing

Reference	56	278680	6285450

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	89.7 MHz

Polarisation	Mixed

Maximum antenna height	30 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 40 W |

Special Condition :-

This specification will have its frequency replaced by 89.9 MHz, described under attachment 18.2, at a date not later than six months after publication of the Gazette notice  that the licence area plan for Sydney has been determined.

Attachment 18.3

LICENCE AREA PLAN : Sydney – December 1999

Category :	Community

General Area Served :	Windsor (NSW)

Service Licence Number :	SL5943

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10008374

Transmitter Site :-

Nominal location :	Hawkesbury City Council Tower  Kurrajong Heights

Australian Map Grid :	Zone	Easting	Northing

Reference	56	279000	6288305

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	89.9 MHz

Polarisation	Mixed

Maximum antenna height	30 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 90°T | 500 W |
| 90°T - 180°T | 50 W |
| 180°T - 270°T | 500 W |
| 270°T - 360°T | 750 W |

Special Condition :-

The service is interference limited, only field strength levels of 66dBuV/m and above will be protected from interference.

This specification will replace the frequency 89.7 MHz, described under attachment 18.2, at a date not later than six months after publication of the Gazette that the licence area plan for Sydney has been determined.