VARIATION TO LICENCE AREA PLAN

Cairns 
(Radio)

Australian Broadcasting Authority
Canberra
 September 2004

© Commonwealth of Australia  September 2004

This work is copyright. Apart from fair dealings for the purpose of private study, research, criticism or review, as permitted by the Copyright Act 1968, no part may be reproduced or transmitted, in any form or by any means or process, without the written permission of the publishers.

Published by the Australian Broadcasting Authority
PO Box 34
Belconnen
ACT 2616

VARIATION TO LICENCE AREA PLAN

CAIRNS – SEPTEMBER 2004

The Australian Broadcasting Authority herby varies the licence area plan (LAP) for radio in the Cairns region of Queensland, determined on 18 October 1997 and subsequently varied on 8 October 1998, under subsection 26(2) of the Broadcasting Services Act 1992.

The variation determines that channel capacity be made available for:

- two national radio services in Cairns and Cairns North;

- two FM transmitters for the 4CCA commercial radio service in Redlynch Valley and Little Mulgrave;

- an FM transmitter for an additional community radio service in Cairns;

- an open narrowcasting service in Port Douglas; and

vary the technical specifications of two commercial radio services in Mossman , as set out below:

| Service Category | Service Licence No. | Technical Specification No. | Frequency | Attach No. | Area Served | Status |
| --- | --- | --- | --- | --- | --- | --- |
| National | N/A | TS10013639 | 96.3 MHz | 1.23 | Cains North | Add |
| National | N/A | TS10013402 | 101.1 MHz | 1.24 | Cairns | Add |
| Commercial | SL1150014 | TS10013339 | 91.5 MHz | 1.25 | Redlynch Valley | Add |
| Commercial | SL1150014 | TS10013395 | 93.5 MHz | 1.26 | Little Mulgrave | Add |
| Community | SL1150829 | TS10013927 | 101.9 MHz | 3.2 | Cairns | Add |
| Open Narrowcasting | N/A | TS10013925 | 106.3 MHz | 1.27 | Port Douglas | Add |
| Commercial | SL1150014 | TS10007487 | 91.7 MHz | 1.5.1 | Mossman | Vary |
| Commercial | SL5742 | TS10007488 | 92.5 MHz | 1.7.1 | Mossman | Vary |

The variation also redefines the licence areas of the existing Cairns commercial radio services, 4IL, 4HOT, 4CCA and 4RGC and the Cairns community radio services 4CCR and 4CIM and defines the licence area of the new Cairns community radio service using 2001 Census boundaries.

The descriptions of the updated licence areas are at Attachments 1.1.1, 2 .1.1 and 3.1.

(Determined by the Australian Broadcasting Authority on 23 September 2004)

Schedules

Each of the Schedules sets out:

- the category of each of the services that are to be available in that area (column one).

- the status of the service (column two) does not form part of the licence area plan decision and is included for reference only.

- the frequency for each transmitter (column three) does not form part of the licence area plan decision and is included for reference only.

- the service licence number which has been allocated to the service, if appropriate (column four);  N/A = Not Applicable, as the Broadcasting Services Act 1992 only requires service licences for Commercial and Community services.

- the transmitter specification number which has been allocated to each transmitter (column five),

- the identifying number of the attachment where the technical specifications for each transmitter are described (column six).  (Each set of technical specifications referred to in the Schedules is an attachment to this determination).

- the general area served by each transmitter (column seven).

Status Column

Licensed:	service is already licensed

Available:	new licence available for allocation

Proposed:	transmitter for existing broadcasting service licence

Alternative:	alternative channel proposed for licensed service

Reserved:	capacity reserved for a national or community service in accordance with Ministerial notification

Frequency Column

The frequency shown is the centre frequency of the channel.

The information in the frequency column is provided as additional information to the licence area plan, and is accurate as at the date of publication.

For ease of reference, the schedule affected by this draft variation to the LAP has been updated to reflect the proposed variation to this plan.

Schedule One of the Cairns LAP prior to this proposed variation is reproduced below.

SCHEDULE ONE

Licence Area Plan : Cairns - (Radio) - October 1997 - as varied October 1998

Licence Area : Cairns RA1

| Service Category | Status | Frequency | Service Licence No | Technical Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Commercial | Licensed | 846 kHz | SL4170 | TS10007435 | 1.2 | Cairns |
| Commercial | Licensed | 954 kHz | SL4170 | TS10007448 | 1.3 | Gordonvale |
| Commercial | Licensed | 103.5 MHz | SL5742 | TS10007433 | 1.4 | Cairns |
| Commercial | Licensed | 91.7 MHz | SL5742 | TS10007487 | 1.5 | Mossman |
| Commercial | Available | 102.7 MHz | SL1150014 | TS10007328 | 1.6 | Cairns |
| Commercial | Available | 92.5 MHz | SL1150014 | TS10007488 | 1.7 | Mossman |
| Commercial | Available | 99.5 MHz | SL1150044 | TS10007855 | 1.8 | Cairns |
| Commercial | Available | 88.5 MHz | SL1150044 | TS0007856 | 1.9 | Mossman |
| National | Licensed | 801 kHz | N/A | TS4046001 | 1.22 | Cairns |
| National | Licensed | 105.9 MHz | N/A | TS10007417 | 1.10 | Cairns |
| National | Licensed | 105.1 MHz | N/A | TS10007418 | 1.11 | Cairns |
| National | Licensed | 105.9 MHz | N/A | TS10007419 | 1.12 | Cairns |
| National | Licensed | 107.5 MHz | N/A | TS10007421 | 1.13 | Cairns |
| National | Licensed | 95.5 MHz | N/A | TS10007422 | 1.14 | Cairns North |
| National | Licensed | 93.9 MHz | N/A | TS10007423 | 1.15 | Cairns North |
| National | Licensed | 97.1 MHz | N/A | TS10007430 | 1.16 | Cairns North |
| National | Licensed | 94.7 MHz | N/A | TS10007586 | 1.17 | Cairns North |
| National | Licensed | 90.1 MHz | N/A | TS10007432 | 1.18 | Mossman |
| National | Licensed | 639 kHz | N/A | TS10007457 | 1.19 | Mossman |
| Open Narrowcasting | Available | 104.3 MHz | N/A | TS10007456 | 1.20.1 | Cairns |
| Open Narrowcasting | Licensed | 90.9 MHz | N/A | TS10007489 | 1.21 | Mossman |

The updated schedule to the LAP affected by this variation is as follows:

Schedule One

Licence Area Plan : Cairns Radio -  September 2004

Licence Area : CAIRNS RA1

| Service Category | Status | Channel/ Frequency | Service Licence No | Transmitter Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Commercial | Licensed | 846 kHz | SL4170 | TS10007435 | 1.2 | Cairns |
|  | Licensed | 954 kHz | SL4170 | TS10007448 | 1.3 | Gordonvale |
| Commercial | Licensed | 103.5 MHz | SL5742 | TS10007433 | 1.4 | Cairns (Mt Yarrabah) |
|  | Licensed | 91.7 MHz | SL5742 | TS10007487 | 1.5.1 | Mossman |
| Commercial | Licensed | 102.7 MHz | SL1150014 | TS10007328 | 1.6 | Cairns (Mt Yarrabah) |
|  | Licensed | 92.5 MHz | SL1150014 | TS10007488 | 1.7.1 | Mossman |
|  | Proposed | 91.5 MHz | SL1150014 | TS10013339 | 1.25 | Redlynch Valley |
|  | Proposed | 93.5 MHz | SL1150014 | TS10013395 | 1.26 | Little Mulgrave |
| Commercial | Licensed | 99.5 MHz | SL1150044 | TS10007855 | 1.8 | Cairns (Mt Yarrabah) |
|  | Licensed | 88.5 MHz | SL1150044 | TS10007856 | 1.9 | Mossman |
| National | Licensed Reserved | 801 kHz | N/A | TS4046001 | 1.22 | Cairns |
| National | Licensed Reserved | 106.7 MHz | N/A | TS10007417 | 1.10 | Cairns |
| National | Licensed Reserved | 105.1 MHz | N/A | TS10007418 | 1.11 | Cairns |
| National | Licensed Reserved | 105.9 MHz | N/A | TS10007419 | 1.12 | Cairns |
| National | Licensed Reserved | 107.5 MHz | N/A | TS10007421 | 1.13 | Cairns |
| National | Proposed Reserved | 101.1 MHz | N/A | TS10013402 | 1.24 | Cairns |
| National | Licensed Reserved | 95.5 MHz | N/A | TS10007422 | 1.14 | Cairns North |
| National | Licensed Reserved | 93.9 MHz | N/A | TS10007423 | 1.15 | Cairns North |
| National | Licensed Reserved | 97.1 MHz | N/A | TS10007430 | 1.16 | Cairns North |
| National | Licensed Reserved | 94.7 MHz | N/A | TS10007586 | 1.17 | Cairns North |
| National | Proposed Reserved | 96.3 MHz | N/A | TS10013639 | 1.23 | Cairns North |
| National | Licensed Reserved | 90.1 MHz | N/A | TS10007432 | 1.18 | Mossman |
| National | Licensed Reserved | 639 kHz | N/A | TS10007457 | 1.19 | Mossman |
| Open Narrowcasting | Licensed | 104.3 MHz | N/A | TS10007456 | 1.20.1 | Cairns (Mt Yarrabah) |
| Open Narrowcasting | Licensed | 90.9 MHz | N/A | TS10007489 | 1.21 | Mossman |
| Open Narrowcasting | Available | 106.3 MHz | N/A | TS10013925 | 1.27 | Port Douglas |

Schedule 3

Draft Licence Area Plan : Cairns Radio -  September 2004

Licence Area : CAIRNS RA3

| Service Category | Status | Channel/ Frequency | Service Licence No | Transmitter Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- | --- |
| Community | Available | 101.9 MHz | SL1150829 | TS10013927 | 3.2 | Cairns |

Attachment	1.1.1

Licence Area - CAIRNS RA1

Commercial Radio Service Licence numbers:  SL4170, SL5742, SL1150014, SL1150044

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 5 August 2001, is:

Area Description

Cairns (C) - Barron (SLA)

Cairns (C) - Central Suburbs (SLA)

Cairns (C) - City (SLA)

Cairns (C) - Mt Whitfield (SLA)

Cairns (C) - Northern Suburbs (SLA)

Cairns (C) - Trinity (SLA)

Cairns (C) - Western Suburbs (SLA)

QLD CD 010803

QLD CD 010804

QLD CD 010805

QLD CD 010806

QLD CD 010807

QLD CD 010808

QLD CD 010810

QLD CD 010901

QLD CD 010902

QLD CD 010903

QLD CD 010904

QLD CD 010905

QLD CD 010906

QLD CD 010907

QLD CD 010908

QLD CD 010909

QLD CD 010910

QLD CD 010911

QLD CD 010912

QLD CD 010913

QLD CD 010914

QLD CD 010915

QLD CD 010916

QLD CD 010917

QLD CD 010918

QLD CD 012002

QLD CD 012003

QLD CD 012004

QLD CD 012005

QLD CD 012007

QLD CD 012008

QLD CD 012011

QLD CD 012012

QLD CD 012206

QLD CD 012217

QLD CD 012301

QLD CD 012302

QLD CD 030302

QLD CD 030303

QLD CD 030304

QLD CD 030311

QLD CD 030312

QLD CD 030313

QLD CD 030314

QLD CD 030316

QLD CD 030317

QLD CD 030318

Note:

Standard terminology used by the Australian Bureau of Statistics:

(C) 	=  	City

(SLA) 	=  	Statistical Local Area

(CD) 	=  	Collection District

Attachment	1.5.1

LICENCE AREA PLAN :	Cairns Radio -  September 2004

Category :	Commercial

General Area Served :	Mossman (QLD)

Service Licence Number :	SL5742

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10007487

Transmitter Site :-

Nominal location :	Crown Castle Mossman Site  Gorge View 	Crescent  MOSSMAN

Australian Map Grid :	Zone	Easting	Northing
Reference	55	325240	8178150

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	91.7 MHz

Polarisation	Mixed

Maximum antenna height	40 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 1 kW |

Attachment	1.7.1

LICENCE AREA PLAN :	Cairns Radio -  September 2004

Category :	Commercial

General Area Served :	Mossman (QLD)

Service Licence Number :	SL1150014

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10007488

Transmitter Site :-

Nominal location :	Crown Castle Mossman Site  Gorge View 	Crescent  MOSSMAN

Australian Map Grid :	Zone	Easting	Northing
Reference	55	325240	8178150

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	92.5 MHz

Polarisation	Mixed

Maximum antenna height	40 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 1 kW |

Attachment	1.23

LICENCE AREA PLAN :	Cairns Radio -  September 2004

Category :	National

General Area Served :	Cairns North (QLD)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10013639

Transmitter Site :-

Nominal location :	Broadcast Site  BUCHAN POINT

Australian Map Grid :	Zone	Easting	Northing
Reference	55	357300	8148900

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	96.3 MHz

Polarisation	Mixed

Maximum antenna height	37 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 118°T | 2.5 kW |
| 118°T - 133°T | 5 kW |
| 133°T - 203°T | 10 kW |
| 203°T - 218°T | 5 kW |
| 218°T - 360°T | 2.5 kW |

Attachment	1.24

LICENCE AREA PLAN :	Cairns Radio -  September 2004

Category :	National

General Area Served :	Cairns (QLD)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10013402

Transmitter Site :-

Nominal location :	Radio Terminal  MT BELLENDEN-KER

Australian Map Grid :	Zone	Easting	Northing
Reference	55	378034	8090678

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	101.1 MHz

Polarisation	Mixed

Maximum antenna height	53 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 100 kW |

Special Conditions :-

Note: The antenna height of this national service has been changed from 50 to 53 metres. Refer to file 1998/5261.

Attachment	1.25

LICENCE AREA PLAN :	Cairns Radio -  September 2004

Category :	Commercial

General Area Served :	Redlynch Valley (QLD)

Service Licence Number :	SL1150014

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10013339

Transmitter Site :-

Nominal location :	NTL Site  Mt Williams Redlynch Valley  	CAIRNS

Australian Map Grid :	Zone	Easting	Northing
Reference	55	360380	8129200

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	91.5 MHz

Polarisation	Mixed

Maximum antenna height	25 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 500 W |

Special Conditions :-

Any transmission in accordance with this technical specification is planned on the basis that it will be protected to a minimum median field strength level of 66 dBuV/m and above against interference from other broadcasting services.

Attachment	1.26

LICENCE AREA PLAN :	Cairns Radio -  September 2004

Category :	Commercial

General Area Served :	Little Mulgrave (QLD)

Service Licence Number :	SL1150014

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10013395

Transmitter Site :-

Nominal location :	Broadcast Planning Site  Goldsborough 	Valley  LITTLE MULGRAVE

Australian Map Grid :	Zone	Easting	Northing
Reference	55	366550	8101950

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	93.5 MHz

Polarisation	Mixed

Maximum antenna height	20 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 500 W |

Special Conditions :-

Any transmission in accordance with this technical specification is planned on the basis that it will be protected to a minimum median field strength level of 66 dBuV/m and above against interference from other broadcasting services.

Attachment	1.27

LICENCE AREA PLAN :	Cairns Radio -  September 2004

Category :	Open Narrowcasting

General Area Served :	Port Douglas (QLD)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10013925

Transmitter Site :-

Nominal location :	Communications Tower  Flagstaff Hill  	PORT DOUGLAS

Australian Map Grid :	Zone	Easting	Northing
Reference	55	336262	8176960

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	106.3 MHz

Polarisation	Mixed

Maximum antenna height	20 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 100 W |

Special Conditions :-

Any transmission in accordance with this technical specification is planned on the basis that it will be protected to a minimum median field strength level of 66 dBuV/m and above against interference from other broadcasting services.

The 'coverage radius' of this transmission is 10 kilometres measured from the nominal transmitter location (AMG co-ordinates: Zone 55; Easting 336262; Northing 8176960)

Attachment	2.1.1

Licence Area - CAIRNS RA2

Community Radio Service Licence numbers:  SL1883, SL1150013

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 5 August 2001, is:

Area Description

Cairns (C) - Central Suburbs (SLA)

Cairns (C) - City (SLA)

Cairns (C) - Mt Whitfield (SLA)

QLD CD 011217

QLD CD 011301

QLD CD 011302

QLD CD 011303

QLD CD 011304

QLD CD 011307

QLD CD 011316

QLD CD 011317

QLD CD 011505

QLD CD 011510

QLD CD 011511

QLD CD 011514

QLD CD 011515

QLD CD 011601

QLD CD 011602

QLD CD 011603

QLD CD 011604

QLD CD 011605

QLD CD 011606

QLD CD 011607

QLD CD 011608

QLD CD 011610

QLD CD 011611

QLD CD 011612

QLD CD 011613

QLD CD 011614

QLD CD 011615

QLD CD 011617

QLD CD 011618

QLD CD 011701

QLD CD 011702

QLD CD 011703

QLD CD 011704

QLD CD 011705

QLD CD 011706

QLD CD 011707

QLD CD 011708

QLD CD 011709

QLD CD 011710

QLD CD 011711

QLD CD 011712

QLD CD 011713

QLD CD 011714

QLD CD 011802

QLD CD 011803

QLD CD 011804

QLD CD 011805

QLD CD 011806

QLD CD 011807

QLD CD 011808

QLD CD 011809

QLD CD 011810

QLD CD 011811

QLD CD 011812

QLD CD 011901

QLD CD 011906

QLD CD 011908

QLD CD 011909

QLD CD 011911

QLD CD 011913

QLD CD 011914

QLD CD 011918

QLD CD 012101

QLD CD 012102

QLD CD 012103

QLD CD 012104

QLD CD 012105

QLD CD 012109

QLD CD 012112

QLD CD 012115

QLD CD 012116

QLD CD 012122

QLD CD 012202

QLD CD 012203

QLD CD 012204

QLD CD 012212

QLD CD 012213

QLD CD 012219

QLD CD 012220

QLD CD 012221

QLD CD 012701

QLD CD 012702

QLD CD 012703

QLD CD 012704

QLD CD 012705

QLD CD 012706

QLD CD 012707

QLD CD 012708

QLD CD 012709

QLD CD 012710

QLD CD 012711

QLD CD 012712

QLD CD 012713

QLD CD 012714

QLD CD 012715

QLD CD 012716

Note:

Standard terminology used by the Australian Bureau of Statistics:

(C) 	=  	City

(SLA) 	=  	Statistical Local Area

(CD) 	=  	Collection District

Attachment	3.1

Licence Area - CAIRNS RA3

Community Radio Service Licence numbers:  SL1150829

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 5 August 2001, is:

Area Description

QLD CD 011001

QLD CD 011002

QLD CD 011006

QLD CD 011007

QLD CD 011008

QLD CD 011208

QLD CD 011209

QLD CD 011211

QLD CD 011212

QLD CD 011408

QLD CD 011409

QLD CD 011514

QLD CD 011601

QLD CD 011602

QLD CD 011603

QLD CD 011604

QLD CD 011605

QLD CD 011606

QLD CD 011607

QLD CD 011608

QLD CD 011610

QLD CD 011611

QLD CD 011612

QLD CD 011613

QLD CD 011614

QLD CD 011615

QLD CD 011616

QLD CD 011617

QLD CD 011618

QLD CD 011619

QLD CD 011620

QLD CD 011701

QLD CD 011702

QLD CD 011703

QLD CD 011704

QLD CD 011705

QLD CD 011706

QLD CD 011707

QLD CD 011708

QLD CD 011709

QLD CD 011710

QLD CD 011711

QLD CD 011712

QLD CD 011713

QLD CD 011714

QLD CD 011801

QLD CD 011802

QLD CD 011804

QLD CD 011805

QLD CD 011806

QLD CD 011807

QLD CD 011808

QLD CD 011809

QLD CD 011810

QLD CD 011811

QLD CD 011901

QLD CD 011902

QLD CD 011903

QLD CD 011904

QLD CD 011905

QLD CD 011906

QLD CD 011907

QLD CD 011908

QLD CD 011909

QLD CD 011912

QLD CD 011913

QLD CD 011914

QLD CD 011915

QLD CD 011916

QLD CD 011919

QLD CD 012012

QLD CD 012124

Note:

Standard terminology used by the Australian Bureau of Statistics:

(CD) 	=  	Collection District

Attachment	3.2

LICENCE AREA PLAN :	Cairns Radio -  September 2004

Category :	Community

General Area Served :	Cairns (QLD)

Service Licence Number :	SL1150829

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10013927

Transmitter Site :-

Nominal location :	Smithfield State High School  	SMITHFIELD

Australian Map Grid :	Zone	Easting	Northing
Reference	55	360890	8140460

Site Tolerance :	Refer to Technical Planning Guidelines

Emission :-

Frequency Band & Mode	VHF-FM

Carrier Frequency :	101.9 MHz

Polarisation	Vertical

Maximum antenna height	30 m

Output Radiation Pattern :-

| Bearing or Sector (Clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 2 kW |