I, William Bruce Byron, Director of Aviation Safety, on behalf of CASA, make this instrument under regulation 207 of the Civil Aviation Regulations 1988.

[Signed Bruce Byron]

Bruce Byron
Director of Aviation Safety and
   Chief Executive Officer

6 June 2007

Civil Aviation Order 20.18 Amendment Order (No. 1) 2007

1	Name of instrument

This instrument is the Civil Aviation Order 20.18 Amendment Order (No. 1) 2007.

2	Commencement

This instrument commences on the day after it is registered.

3	Amendment of Civil Aviation Order 20.18

Schedule 1 amends Civil Aviation Order 20.18.

Schedule 1	Amendment

[1]	After subsection 9A

insert

9B	Directions relating to carriage and use of automatic dependent surveillance – broadcast equipment

9B.1	In this subsection and Appendix XI:

ADS-B means automatic dependent surveillance – broadcast.

aircraft address means a unique 24-bit aircraft address code assigned to an aircraft in accordance with Annex 10 to the Chicago Convention.

approved equipment configuration means the requirements:

(a)	set out in Appendix XI; or

(b)	approved in writing by CASA and published from time to time in an Advisory Circular.

Note    Equipment configurations approved by CASA are published in Appendix D of Advisory Circular 21-45.

9B.2	ADS-B transmitting equipment carried by an aircraft for operational use in Australia must comply with an approved equipment configuration.

9B.3	If ADS-B transmitting equipment carried by an aircraft does not comply with an approved equipment configuration, it must be deactivated before flight in Australia.

9B.4	ADS-B transmitting equipment carried by an aircraft for operational use in Australia must transmit:

(a)	a flight identification that corresponds exactly to the aircraft identification mentioned on the flight notification filed with ATS for the flight; or

(b)	if no flight notification is filed for the flight — a flight identification that is:

(i)	for an aircraft registered on the Australian Civil Aircraft Register and operating wholly within Australian territory — the aircraft’s registration mark; or

(ii)	for an Australian aircraft registered by a recreational aircraft administrative organisation — in accordance with the organisation’s operations manual; or

(iii)	in any other case — the aircraft’s nationality or common mark and registration mark without any hyphen included; or

- another flight identification directed or approved by ATC.

[2]	After Appendix X

insert

Appendix XI	ADS-B transmitting equipment

1	ADS-B transmitting equipment must meet:

(a)	TSO-C166 as in force on 20 September 2004; or

(b)	TSO-C166a as in force on 21 December 2006; or

(c)	ATSO-C1004 as in force on 2 October 2003; or

(d)	ATSO-C1005 as in force on 22 December 2004; or

(e)	another standard acceptable to CASA.

2	On and after 28 June 2012, the geographical position transmitted by the equipment must be determined by:

(a)	a GNSS receiver that meets TSO-C145a or TSO-C146a as in force on 19 September 2002; or

(b)	another system acceptable to CASA for this purpose.

3	The pressure altitude transmitted by the equipment must be determined by:

(a)	a barometric encoder that meets:

(i)	TSO-C88a as in force on 18 August 1983; or

(ii)	ETSO-C88a as in force on 24 October 2003; or

(b)	another system acceptable to CASA for this purpose.

4	The equipment must:

(a)	allow the pilot to activate and deactivate it during flight; and

(b)	transmit the current aircraft address.

Note   The requirement in paragraph 4 (a) is met if the equipment has a cockpit control that enables the pilot to turn the ADS-B transmissions on and off.