I, WILLIAM BRUCE BYRON, Director of Aviation Safety, on behalf of CASA, make this instrument under regulation 139.015 of the Civil Aviation Safety Regulations 1998.

[Signed Bruce Byron]

Bruce Byron
Director of Aviation Safety and
   Chief Executive Officer

2 April 2008

Manual of Standards Part 139 Amendment (No. 2) 2008

1	Name of instrument

This instrument is the Manual of Standards Part 139 Amendment (No. 2) 2008.

2	Commencement

This instrument commences on the day after it is registered.

3	Amendment of the Manual of Standards Part 139

Schedule 1 amends Manual of Standards Part 139 — Aerodromes.

Schedule 1	Amendments

[1]	Paragraph 2.1.6.2 (Table 2.1-2)

substitute

| AEROPLANE TYPE | REF CODE | AEROPLANE CHARACTERISTICS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ARFL (m) | Wing-span (m) | OMGWS (m) | Length (m) | MTOW (kg) | TP (kPa) |
| DHC2 Beaver | 1A | 381 | 14.6 | 3.3 | 10.3 | 2490 | 240 |
| Beechcraft: 58 (Baron) 100 | 1A 1A | 401 628 | 11.5 14.0 | 3.1 4.0 | 9.1 12.2 | 2449 5352 | 392 - |
| Britten Norman Islander | 1A | 353 | 14.9 | 4.0 | 10.9 | 2850 | 228 |
| Cessna: 172 206 310 404 | 1A 1A 1A 1A | 272 274 518 721 | 10.9 10.9 11.3 14.1 | 2.7 2.6 3.7 4.3 | 8.2 8.6 9.7 12.1 | 1066 1639 2359 3810 | - - 414 490 |
| Partenavia P68 | 1A | 230 | 12.0 | 2.6 | 9.4 | 1960 | - |
| Piper: PA 31 (Navajo) PA 34 | 1A 1A | 639 378 | 12.4 11.8 | 4.3 3.4 | 9.9 8.7 | 2950 1814 | 414 - |
| Beechcraft 200 | 1B | 592 | 16.6 | 5.6 | 13.3 | 5670 | 735 |
| Cessna: 208A (Caravan) 402C 441 | 1B 1B 1B | 296 669 544 | 15.9 13.45 15.1 | 3.7 5.6 4.6 | 11.5 11.1 11.9 | 3310 3107 4468 | - 490 665 |
| DHC 6 Twin Otter | 1B | 695 | 19.8 | 4.1 | 15.8 | 5670 | 220 |
| Dornier 228-200 | 1B | 525 | 17.0 | 3.6 | 16.6 | 5700 | - |
| DHC-7 | 1C | 689 | 28.4 | 7.8 | 24.6 | 19505 | 620 |
| DHC-5E | 1D | 290 | 29.3 | 10.2 | 24.1 | 22316 | - |
| Lear Jet 28/29 | 2A | 912 | 13.4 | 2.5 | 14.5 | 6804 | 793 |
| Beechcraft 1900 | 2B | 1098 | 16.6 | 5.8 | 17.6 | 7530 | - |
| CASA C-212 | 2B | 866 | 20.3 | 3.5 | 16.2 | 7700 | 392 |
| Embraer EMB110 | 2B | 1199 | 15.3 | 4.9 | 15.1 | 5670 | 586 |
| Metro II | 2B | 800 | 14.1 | 5.4 | 18.1 | 5670 | 740 |
| Metro III | 2B | 991 | 17.37 | 5.4 | 18.1 | 6577 | 740 |
| ATR 42-200 | 2C | 1010 | 24.6 | 4.9 | 22.7 | 16150 | 728 |
| Cessna 550 | 2C | 912 | 15.8 | 6.0 | 14.4 | 6033 | 700 |
| DHC-8: 100 300 | 2C 2C | 948 1122 | 25.9 27.4 | 8.5 8.5 | 22.3 25.7 | 15650 18642 | 805 805 |
| Lear Jet 55 | 3A | 1292 | 13.4 | 2.5 | 16.8 | 9298 | - |
| IAI Westwind 2 | 3A | 1495 | 13.7 | 3.7 | 15.9 | 10660 | 1000 |
| BAe 125-400 | 3B | 1713 | 15.7 | 3.3 | 15.5 | 12480 | 1007 |
| Canadair: CL600 CRJ-200 | 3B 3B | 1737 1527 | 18.9 21.21 | 4.0 4.0 | 20.9 26.77 | 18642 21523 | 1140 1117 |
| Cessna 650 | 3B | 1581 | 16.3 | 3.6 | 16.9 | 9979 | 1036 |
| Dassault-Breguet: Falcon 900 | 3B | 1515 | 19.3 | 5.3 | 20.2 | 20640 | 1300 |
| Embraer EMB 145 | 3B | 1500 | 20 | 4.8 | 29.9 | 19200 | - |
| Fokker F28-2000 | 3B | 1646 | 23.6 | 5.8 | 29.6 | 29480 | 689 |
| Metro 23 | 3B | 1341 | 17.4 | 5.4 | 18.1 | 7484 | 742 |
| Shorts SD3-60 | 3B | 1320 | 22.8 | 4.6 | 21.6 | 11793 | 758 |
| Bae: Jetstream 31 Jetstream 41 146-200 146-300 | 3C 3C 3C 3C | 1440 1500 1615 1615 | 15.9 18.3 26.3 26.3 | 6.2 - 5.5 5.5 | 14.4 19.3 26.2 31.0 | 6950 10433 42185 44225 | 448 - 1138 945 |
| Bombadier Global Express | 3C | 1774 | 28.7 | 4.9 | 30.3 | 42410 | - |
| Embraer: EMB 120 EMB 170 | 3C 3C | 1420 1600 | 19.8 26.0 | 7.3 5.8 | 20.0 29.90 | 11500 37200 | 828 940 |
| Fokker: F27-500 F28-4000 F50 F100 | 3C 3C 3C 3C | 1670 1640 1760 1695 | 29.0 25.1 29.0 28.1 | 7.9 5.8 8.0 5.0 | 25.1 29.6 25.2 35.5 | 20412 32205 20820 44450 | 540 779 552 920 |
| SAAB SF-340 | 3C | 1220 | 21.4 | 7.5 | 19.7 | 12371 | 655 |
| Airbus A300 B2 | 3D | 1676 | 44.8 | 10.9 | 53.6 | 142000 | 1241 |
| Bombardier Dash 8 – Q400 | 3D | 1354 | 28.4 | 9.6 | 32.8 | 29000 | 1020 |
| Airbus A320-200 | 4C | 2058 | 33.9 | 8.7 | 37.6 | 72000 | 1360 |
| Boeing: B717-200 B737-200 B737-300 B737-400 B737-800 Embraer EMB 190 | 4C 4C 4C 4C 4C 4C | 2130 2295 2749 2499 2256 2110 | 28.4 28.4 28.9 28.9 35.8 28.72 | 6.0 6.4 6.4 6.4 6.4 6.6 | 37.8 30.6 30.5 36.5 39.5 36.24 | 51710 52390 61230 63083 70535 51800 | - 1145 1344 1400 - 1080 |
| McDonnell Douglas: DC9-30 DC9-80/MD80 | 4C 4C | 2134 2553 | 28.5 32.9 | 6.0 6.2 | 37.8 45.1 | 48988 72575 | - 1390 |
| Airbus: A300-600 A310-200 | 4D 4D | 2332 1845 | 44.8 43.9 | 10.9 10.9 | 54.1 46.7 | 165000 132000 | 1260 1080 |
| Boeing: B707-300 B757-200 B767-200ER B767-300ER | 4D 4D 4D 4D | 3088 2057 2499 2743 | 44.4 38.0 47.6 47.6 | 7.9 8.7 10.8 10.8 | 46.6 47.3 48.5 54.9 | 151315 108860 156500 172365 | 1240 1172 1310 1310 |
| McDonnell Douglas: DC8-63 DC10-30 | 4D 4D | 3179 3170 | 45.2 50.4 | 7.6 12.6 | 57.1 55.4 | 158757 251744 | 1365 1276 |
| Lockheed: L1011-100/200 | 4D | 2469 | 47.3 | 12.8 | 54.2 | 211378 | 1207 |
| McDonnell Douglas MD11 | 4D | 2207 | 51.7 | 12.0 | 61.2 | 273289 | 1400 |
| Airbus: A330-200 A330-300 A340-300 A340-500 A340-600 | 4E 4E 4E 4E 4E | 2713 2560 2200 3275 3185 | 60.3 60.3 60.3 63.70 63.70 | 12.0 12.0 12.0 12.0 12.0 | 59.0 63.6 63.7 67.80 75.30 | 230000 230000 253500 368000 365000 | 1400 1400 1400 1400 1400 |
| Boeing: B747-SP B747-300 B747-400 B777-200 B777-300 | 4E 4E 4E 4E 4E | 2710 3292 3383 2500 3140 | 59.6 59.6 64.9 60.9 60.93 | 12.4 12.4 12.4 12.8 12.6 | 56.3 70.4 70.4 63.73 73.86 | 318420 377800 394625 287800 299370 | 1413 1323 1410 1400 1400 |
| Airbus A380-800 | 4F | 3350 | 79.8 | 14.3 | 72.7 | 560000 | 1400 |