I, terence lindsay farquharson, Acting Director of Aviation Safety, on behalf of CASA, make this instrument under regulation 90.020 of the Civil Aviation Safety Regulations 1998.

[Signed T. Farquharson]

Terry Farquharson
Acting Director of Aviation Safety

12 October 2011

- Manual of Standards Part 90 Amendment Instrument 2011 (No. 1)

- 1	Name of instrument

This instrument is the Manual of Standards Part 90 Amendment Instrument 2011 (No. 1).

- 2	Commencement

This instrument commences on the day after registration.

3	Amendment of Manual of Standards Part 90 Instrument 2010

Manual of Standards Part 90 Instrument 2010 is amended as set out in Schedule 1.

Schedule 1	Amendments

[1]	Section 1

repeal

[2]	Section 2

repeal

[3]	Section 3

repeal

[4]	Schedule 1, heading

omit

[5]	Schedule 1

omit

Additional airworthiness standards

[6]	Part 1, General

before

1	Definitions

insert

1A	Name of instrument

This instrument is the Part 90 Manual of Standards (MOS).

[7]	Schedule 1, subsection 31.4

substitute

31.4	If the application for the type certificate for a transport category aeroplane was filed before 1 May 1972, for each Type II or larger passenger emergency exit with a locking mechanism released by rotary motion of a handle, the instructions for opening must be shown by:

(a)	a red arrow with a shaft at least 19 mm wide and a head twice the width of the shaft, extending along at least 70 degrees of arc at a radius approximately equal to three quarters of the handle length; and

(b)	the word OPEN in red capital letters at least 25 mm high on a contrasting background placed horizontally adjacent to the arrowhead to show the open position of the handle.