Radiocommunications (Digital Radio Channels – Queensland) Plan Variation 2020 (No. 1)

The Australian Communications and Media Authority makes the following instrument under section 44A of the Radiocommunications Act 1992.

Dated: 10 December 2020

Fiona Cameron

[signed]

Member

Creina Chapman

[signed]

Member/General Manager

Australian Communications and Media Authority

1  Name

This is the Radiocommunications (Digital Radio Channels – Queensland) Plan Variation 2020 (No. 1).

2  Commencement

This instrument commences at the start of the day after the day it is registered on the Federal Register of Legislation.

Note:	The Federal Register of Legislation may be accessed free of charge at www.legislation.gov.au.

3  Authority

This instrument is made under section 44A of the Radiocommunications Act 1992.

4  Amendments

The instrument that is specified in Schedule 1 is amended as set out in the applicable items in that Schedule.

Schedule 1—Amendments

Radiocommunications (Digital Radio Channels – Queensland) Plan 2007 (F2007L04664)

1  Section 3

Insert:

GDA 94 means the geodetic datum designated as the “Geocentric Datum of Australia (GDA)”, gazetted in the Commonwealth of Australia Gazette No. GN 35 on 6 September 1995, as existing on that date.

2  Section 3, definition of technical planning guidelines

Omit ‘, as in force from time to time’

3  After section 3A

Insert:

## 3B  References to other instruments

In this Plan, unless the contrary intention appears:

(a)	a reference to another legislative instrument is a reference to that other legislative instrument as in force from time to time; and

(b)	a reference to any other kind of instrument or writing is a reference to that other instrument or writing as in force or existing from time to time.

Note 1:	For references to Commonwealth Acts, see section 10 of the Acts Interpretation Act 1901; and see also subsection 13(1) of the Legislation Act 2003 for the application of the Acts Interpretation Act 1901 to legislative instruments.

Note 2:	All Commonwealth Acts and legislative instruments are registered on the Federal Register of Legislation and are accessible free of charge.

Note 3:	See section 314A of the Act.

4  Section 6

Omit ‘determined by the technical planning guidelines’, insert ‘specified in the licence’

5  Section 6, note to the section

Repeal the note.

6  At the end of Attachment 1.3 to Schedule 1

Add:

Schedule 2	Gold Coast RA1

(subsection 4 (1))

Designated BSA radio area

Gold Coast RA1

Table 1	Frequency channels

| Column 1 Multiplex Name | Column 2 Frequency block | Column 3 Reserved frequency block | Column 4 Centre frequency (MHz) | Column 5 Category | Column 6 Technical Specification Number | Column 7 Technical Specifications (Attachment number) |
| --- | --- | --- | --- | --- | --- | --- |
| Gold Coast 1 | 9D | No | 208.064 | 1 | TS12000073 | 2.1 |
| Gold Coast 2 | 9D | No | 208.064 | 1 | TS12000792 | 2.2 |
| Gold Coast 3 | 8B | Yes | 197.648 | 3 | TS12000074 | 2.3 |
| Gold Coast 4 | 8B | Yes | 197.648 | 3 | TS12000793 | 2.4 |

Table 2	Number of licences to be issued

| Column 1 Licence Category | Column 2 Number of licences |
| --- | --- |
| Category 1 | 1 |
| Category 2 | 0 |
| Category 3 | 1 |

Attachment 2.1	Multiplex Gold Coast 1

| Column 1 Technical specification |  | Column 2 Details |
| --- | --- | --- |
| Category |  | 1 |
| General Area Served |  | Gold Coast |
| Mode |  | DAB |
| Specification number |  | TS12000073 |
| Transmitter Site |  |  |
| Nominal Location |  | GCT Mt Tamborine Site 103 m Tower Golf Course Road MOUNT TAMBORINE |
| GDA 94 |  | Latitude: -27.969604 Longitude: 153.213366 |
| Site Tolerance |  | Refer to technical planning guidelines |
| Emission |  |  |
| Frequency Band |  | VHF |
| Centre Frequency |  | 208.064 MHz (Frequency Block 9D) |
| Polarisation |  | Vertical |
| Maximum antenna height |  | 80 m |
| Output Radiation Pattern |  |  |
| Bearing or sector (clockwise direction) |  | Maximum ERP |
| 0 °T – 10 °T |  | 5 kW |
| 10 °T – 25 °T |  | 10 kW |
| 25 °T – 180 °T |  | 25 kW |
| 180 °T – 195 °T |  | 10 kW |
| 195 °T – 210 °T |  | 5 kW |
| 210 °T – 235 °T |  | 2.5 kW |
| 235 °T – 340 °T |  | 625 W |
| 340 °T – 350 °T |  | 1.25 kW |
| 350 °T – 360 °T |  | 2.5 kW |

Attachment 2.2	Multiplex Gold Coast 2

| Column 1 Technical specification |  | Column 2 Details |
| --- | --- | --- |
| Category |  | 1 |
| General Area Served |  | Gold Coast |
| Mode |  | DAB |
| Specification number |  | TS12000792 |
| Transmitter Site |  |  |
| Nominal Location |  | 40m tower NRN Broadcast Site Bilbrough Lookout SPRINGBROOK |
| GDA 94 |  | Latitude: -28.234736 Longitude: 153.288940 |
| Site Tolerance |  | Refer to technical planning guidelines |
| Emission |  |  |
| Frequency Band |  | VHF |
| Centre Frequency |  | 208.064 MHz (Frequency Block 9D) |
| Polarisation |  | Vertical |
| Maximum antenna height |  | 40 m |
| Output Radiation Pattern |  |  |
| Bearing or sector (clockwise direction) |  | Maximum ERP |
| 0 °T – 105 °T |  | 500 W |
| 105 °T – 120 °T |  | 250 W |
| 120 °T – 135 °T |  | 125 W |
| 135 °T – 305 °T |  | 50 W |
| 305 °T – 320 °T |  | 125 W |
| 320 °T – 335 °T |  | 250 W |
| 335 °T – 360 °T |  | 500 W |

Attachment 2.3	Multiplex Gold Coast 3

| Column 1 Technical specification |  | Column 2 Details |
| --- | --- | --- |
| Category |  | 3 |
| General Area Served |  | Gold Coast |
| Mode |  | DAB |
| Specification number |  | TS12000074 |
| Transmitter Site |  |  |
| Nominal Location |  | GCT Mt Tamborine Site 103 m Tower Golf Course Road MOUNT TAMBORINE |
| GDA 94 |  | Latitude: -27.969604 Longitude: 153.213366 |
| Site Tolerance |  | Refer to technical planning guidelines |
| Emission |  |  |
| Frequency Band |  | VHF |
| Centre Frequency |  | 197.648 MHz (Frequency Block 8B) |
| Polarisation |  | Vertical |
| Maximum antenna height |  | 80 m |
| Output Radiation Pattern |  |  |
| Bearing or sector (clockwise direction) |  | Maximum ERP |
| 0 °T – 10 °T |  | 5 kW |
| 10 °T – 25 °T |  | 10 kW |
| 25 °T – 180 °T |  | 25 kW |
| 180 °T – 195 °T |  | 10 kW |
| 195 °T – 210 °T |  | 5 kW |
| 210 °T – 235 °T |  | 2.5 kW |
| 235 °T – 340 °T |  | 625 W |
| 340 °T – 350 °T |  | 1.25 kW |
| 350 °T – 360 °T |  | 2.5 kW |

Attachment 2.4	Multiplex Gold Coast 4

| Column 1 Technical specification |  | Column 2 Details |
| --- | --- | --- |
| Category |  | 3 |
| General Area Served |  | Gold Coast |
| Mode |  | DAB |
| Specification number |  | TS12000793 |
| Transmitter Site |  |  |
| Nominal Location |  | 40m tower NRN Broadcast Site Bilbrough Lookout SPRINGBROOK |
| GDA 94 |  | Latitude: -28.234736 Longitude: 153.288940 |
| Site Tolerance |  | Refer to technical planning guidelines |
| Emission |  |  |
| Frequency Band |  | VHF |
| Centre Frequency |  | 197.648 MHz (Frequency Block 8B) |
| Polarisation |  | Vertical |
| Maximum antenna height |  | 40 m |
| Output Radiation Pattern |  |  |
| Bearing or sector (clockwise direction) |  | Maximum ERP |
| 0 °T – 105 °T |  | 500 W |
| 105 °T – 120 °T |  | 250 W |
| 120 °T – 135 °T |  | 125 W |
| 135 °T – 305 °T |  | 50 W |
| 305 °T – 320 °T |  | 125 W |
| 320 °T – 335 °T |  | 250 W |
| 335 °T – 360 °T |  | 500 W |