Radiocommunications (Low Interference Potential Devices) Class Licence Variation 2022 (No. 1)

The Australian Communications and Media Authority makes the following variation under subsection 132(1) of the Radiocommunications Act 1992.

Dated: 3 March 2022

James Cameron

[signed]

Member

Cathy Rainsford

[signed]

Member/General Manager

Australian Communications and Media Authority

1  Name

This is the Radiocommunications (Low Interference Potential Devices) Class Licence Variation 2022 (No. 1).

2  Commencement

This instrument commences at the start of the day after the day it is registered on the Federal Register of Legislation.

Note:	The Federal Register of Legislation may be accessed free of charge at www.legislation.gov.au.

3  Authority

This instrument is made under subsection 132(1) of the Radiocommunications Act 1992.

4  Variations

The instrument that is specified in Schedule 1 is varied as set out in the applicable items in that Schedule.

Schedule 1—Variations

Radiocommunications (Low Interference Potential Devices) Class Licence 2015 (F2015L01438)

1  Schedule 1 (table item 25, column 2, paragraphs (b) and (c))

Repeal the paragraphs, substitute:

(b)	43–44, with a carrier frequency of:

(i)	43.05;

(ii)	43.15;

(iii)	43.25;

(iv)	43.35; or

(v)	43.45.

2  Schedule 1 (after table item 63)

Insert:

| 63AA | Radio Local Area Network transmitters | 5925–6425 | 250 mW | (a) The transmitter must only be used indoors. (b) The power spectral density of the transmitter must not exceed 12.5 mW EIRP per MHz. (c) Contention-based protocols for multiple access, such as Carrier Sense Multiple Access (CSMA) or Multiple Access Collision Avoidance (MACA), must be implemented. |
| --- | --- | --- | --- | --- |
| 63AB | Radio Local Area Network transmitters | 5925–6425 | 25 mW | (a) The power spectral density of the transmitter must not exceed 1.25 mW EIRP per MHz. (b) Contention-based protocols for multiple access, such as Carrier Sense Multiple Access (CSMA) or Multiple Access Collision Avoidance (MACA), must be implemented. |