I, Philippa Jillian spence, Director of Aviation Safety, on behalf of CASA, make this instrument under regulation 5 of the Civil Aviation Regulations 1988 and regulations 11.160, 11.205 and 11.245 of the Civil Aviation Safety Regulations 1998.

[Signed P. Spence]

Pip Spence
Director of Aviation Safety

15 February 2024

# Civil Aviation Order 95.55 Amendment Instrument 2024 (No. 1)

1	Name of instrument

This instrument is Civil Aviation Order 95.55 Amendment Instrument 2024 (No. 1).

2	Commencement

This instrument commences on the day after it is registered.

3	Amendment of Civil Aviation Order 95.55

Schedule 1 amends Civil Aviation Order 95.55 (Exemptions from CAR and CASR — Certain Light Sport Aircraft, Lightweight Aeroplanes and Ultralight Aeroplanes) Instrument 2021.

Schedule 1	Amendments

[1]	Subsection 5, the Note

after

These include:

insert (in their respective alphabetical positions)

modification/repair design approval, type certificated,

[2]	Paragraphs 8.7 and 8.8

repeal and substitute

8.7	A person must not operate a type certificated ultralight aeroplane that has been repaired, or modified to differ from its type design, unless the repair or modification is:

(a)	designed in accordance with 1 of the following:

(i)	an approval under regulation 35 of CAR, as in force before 27 June 2011;

(ii)	a modification/repair design approval;

(iii)	an approval mentioned in regulation 21.475 of CASR;

(iv)	an approval under regulation 21.465 or 21.470 of CASR;

(v)	a CASA specification in or under an airworthiness directive;

(vi)	a specification in the aeroplane’s approved maintenance data;

(vii)	an RAAus approval in accordance with a process for the approval of modifications and repairs specified in the RAAus exposition — but only if:

(A)	RAAus is the relevant sport aviation body; and

(B)	a certificate of airworthiness under regulation 21.176 of CASR is not in force for the aeroplane (other than because of the operation of paragraph 21.181 (4) (a) of CASR); and

(b)	carried out in accordance with the approval or the specified design.

Note   Sub-subparagraph 8.7 (a) (vii) does not apply to the modification or repair of a type certificated ultralight aeroplane if a certificate of airworthiness under regulation 21.176 of CASR is in force or would be in force but for the operation of paragraph 21.181 (4) (a) of CASR.

8.8	A person must not operate a type certificated lightweight aeroplane that has been repaired, or modified to differ from its type design, unless the repair or modification is:

(a)	designed in accordance with:

(i)	an approval under regulation 35 of CAR, as in force before 27 June 2011; or

(ii)	a modification/repair design approval; or

(iii)	an approval mentioned in regulation 21.475 of CASR; or

(iv)	an approval under regulation 21.465 or 21.470 of CASR; or

(v)	a CASA specification in or under an airworthiness directive; or

(vi)	a specification in the aeroplane’s approved maintenance data; and

(b)	carried out in accordance with the approved or specified design.

[3]	Paragraph 8.10

omit

ultralight aeroplane (first occurring)

insert

ultralight aeroplane (other than a type certificated ultralight aeroplane)