I, PHILIPPA JILLIAN SPENCE, Director of Aviation Safety, on behalf of CASA, make this instrument under regulation 172.022 of the Civil Aviation Safety Regulations 1998.

[Signed P. Spence]

Pip Spence

Director of Aviation Safety

22 April 2025

Manual of Standards Part 172 Amendment Instrument 2025 (No. 1)

1	Name of instrument

This instrument is the Manual of Standards Part 172 Amendment Instrument 2025 (No. 1).

2	Commencement

This instrument commences on the day after it is registered.

3	Amendment of the Manual of Standards Part 172

Schedule 1 amends the Manual of Standards Part 172—Air Traffic Services.

Schedule 1	Amendment

[1]	After subsection 10.4.1.2

insert

### 10.4.2	Requirements and Procedures for Independent Parallel Approaches

#### 10.4.2.1	In this section, ICAO Doc.4444 has the meaning given by section 3.02.

#### 10.4.2.2	Despite paragraph 1.1.6.1 (f) and subsection 10.1.1.1, paragraph 6.7.3.2.4 of ICAO Doc. 4444, and any successor provision with a different paragraph number, as in force from time to time, does not apply in relation to independent parallel approaches.

#### 10.4.2.3	Independent parallel approaches may be conducted to parallel runways if:

##### (a)	when vectoring an aircraft, the final vector:

###### (i)	enables the aircraft to intercept the final approach course or track at an angle not greater than 30 degrees; and

###### (ii)	provides at least 1 NM straight flight before the interception; and

##### (b)	the aircraft’s descent to the intercept altitude of the appropriate glide path or vertical path for the selected instrument approach procedure is cleared soon enough to provide a period of level flight to dissipate excess aircraft speed.